| modality_group | id | experience | modality | feature_type | feature_dim | dim_reduction | Features |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ECG | 205 | ECG | ECG | hybrid | not coded | no | Instantaneous Frequency (IF) and spectral entropy (SE) |
| ECG | 253 |  | ECG | hybrid | not coded | no | Autocorrelation (AC) and Discrete Cosine Transform (DCT): selection of the considered signal, computation of autocorrelation coefficients with different delays, transformation of autocorrelation coefficients vector using the DCT transform and then using the transformed vector as new feature input for the classifier. |
| ECG | 278 | S1 idle - S1 touchpad ECG | ECG | handcrafted | not coded | no | partially fiducial approach using the waveform amplitude values |
| ECG | 278 | S1 idle - S1 typing 2 ECG | ECG | handcrafted | not coded | no | partially fiducial approach using the waveform amplitude values |
| ECG | 278 | S1 idle - S1 typing1 ECG | ECG | handcrafted | not coded | no | partially fiducial approach using the waveform amplitude values |
| ECG | 278 | S1 idle - S2 idle ECG | ECG | handcrafted | not coded | no | partially fiducial approach using the waveform amplitude values |
| ECG | 330 | Apple watch DT:ACC | ECG | handcrafted | medium | no | 50 autocorrelation coefficients (ACCs) extracted from rectangular windows of length 400 ms centered around the R-Peak |
| ECG | 330 | Apple watch DT:MFCC | ECG | handcrafted | medium | no | 20 features of Mel-Frequency Cepstral Coefficients (MFCCs) extracted from rectangular windows of length 400 ms centered around the R-Peak |
| ECG | 330 | Apple watch DT:QRS | ECG | handcrafted | not coded | no | QRS features: window size of 200-ms, enclosing only the Q, R, and S waves, while excluding the P and T peaks. |
| ECG | 330 | Apple watch KNN:ACC | ECG | handcrafted | medium | no | 50 autocorrelation coefficients (ACCs) extracted from rectangular windows of length 400 ms centered around the R-Peak |
| ECG | 330 | Apple watch KNN:MFCC | ECG | handcrafted | medium | no | 20 features of Mel-Frequency Cepstral Coefficients (MFCCs) extracted from rectangular windows of length 400 ms centered around the R-Peak |
| ECG | 330 | Apple watch KNN:QRS | ECG | handcrafted | not coded | no | QRS features: window size of 200-ms, enclosing only the Q, R, and S waves, while excluding the P and T peaks. |
| ECG | 330 | Apple watch LDA:ACC | ECG | handcrafted | medium | no | 50 autocorrelation coefficients (ACCs) extracted from rectangular windows of length 400 ms centered around the R-Peak |
| ECG | 330 | Apple watch LDA:MFCC | ECG | handcrafted | medium | no | 20 features of Mel-Frequency Cepstral Coefficients (MFCCs) extracted from rectangular windows of length 400 ms centered around the R-Peak |
| ECG | 330 | Apple watch LDA:QRS | ECG | handcrafted | not coded | no | QRS features: window size of 200-ms, enclosing only the Q, R, and S waves, while excluding the P and T peaks. |
| ECG | 330 | Apple watch LR:ACC | ECG | handcrafted | medium | no | 50 autocorrelation coefficients (ACCs) extracted from rectangular windows of length 400 ms centered around the R-Peak |
| ECG | 330 | Apple watch LR:MFCC | ECG | handcrafted | medium | no | 20 features of Mel-Frequency Cepstral Coefficients (MFCCs) extracted from rectangular windows of length 400 ms centered around the R-Peak |
| ECG | 330 | Apple watch LR:QRS | ECG | handcrafted | not coded | no | QRS features: window size of 200-ms, enclosing only the Q, R, and S waves, while excluding the P and T peaks. |
| ECG | 330 | Apple watch RBF-SVM:ACC | ECG | handcrafted | medium | no | 50 autocorrelation coefficients (ACCs) extracted from rectangular windows of length 400 ms centered around the R-Peak |
| ECG | 330 | Apple watch RBF-SVM:MFCC | ECG | handcrafted | medium | no | 20 features of Mel-Frequency Cepstral Coefficients (MFCCs) extracted from rectangular windows of length 400 ms centered around the R-Peak |
| ECG | 330 | Apple watch RBF-SVM:QRS | ECG | handcrafted | not coded | no | QRS features: window size of 200-ms, enclosing only the Q, R, and S waves, while excluding the P and T peaks. |
| ECG | 330 | Apple watch RF:ACC | ECG | handcrafted | medium | no | 50 autocorrelation coefficients (ACCs) extracted from rectangular windows of length 400 ms centered around the R-Peak |
| ECG | 330 | Apple watch RF:MFCC | ECG | handcrafted | medium | no | 20 features of Mel-Frequency Cepstral Coefficients (MFCCs) extracted from rectangular windows of length 400 ms centered around the R-Peak |
| ECG | 330 | Apple watch RF:QRS | ECG | handcrafted | not coded | no | QRS features: window size of 200-ms, enclosing only the Q, R, and S waves, while excluding the P and T peaks. |
| ECG | 330 | Apple watch SVM:ACC | ECG | handcrafted | medium | no | 50 autocorrelation coefficients (ACCs) extracted from rectangular windows of length 400 ms centered around the R-Peak |
| ECG | 330 | Apple watch SVM:MFCC | ECG | handcrafted | medium | no | 20 features of Mel-Frequency Cepstral Coefficients (MFCCs) extracted from rectangular windows of length 400 ms centered around the R-Peak |
| ECG | 330 | Apple watch SVM:QRS | ECG | handcrafted | not coded | no | QRS features: window size of 200-ms, enclosing only the Q, R, and S waves, while excluding the P and T peaks. |
| ECG | 36 | Hexoskin E30 DT | ECG | handcrafted | medium | no | Fiducial features, manhattan- and euclidian distances: Qamp, Ramp, Samp, Tamp, QQman, RRman, SSman, TTman, QQeuc, RReuc, SSeuc, TTeuc, Manhattan distances of QR, ST, RS, RT, QS, QT, RST, and QRS for 1/2/3 peaks |
| ECG | 36 | Hexoskin E30 NB | ECG | handcrafted | medium | no | Fiducial features, manhattan- and euclidian distances: Qamp, Ramp, Samp, Tamp, QQman, RRman, SSman, TTman, QQeuc, RReuc, SSeuc, TTeuc, Manhattan distances of QR, ST, RS, RT, QS, QT, RST, and QRS for 1/2/3 peaks |
| ECG | 36 | Hexoskin E40 DT | ECG | handcrafted | medium | no | Fiducial features, manhattan- and euclidian distances: Qamp, Ramp, Samp, Tamp, QQman, RRman, SSman, TTman, QQeuc, RReuc, SSeuc, TTeuc, Manhattan distances of QR, ST, RS, RT, QS, QT, RST, and QRS for 1/2/3 peaks |
| ECG | 36 | Hexoskin E40 NB | ECG | handcrafted | medium | no | Fiducial features, manhattan- and euclidian distances: Qamp, Ramp, Samp, Tamp, QQman, RRman, SSman, TTman, QQeuc, RReuc, SSeuc, TTeuc, Manhattan distances of QR, ST, RS, RT, QS, QT, RST, and QRS for 1/2/3 peaks |
| ECG | 36 | Hexoskin E50 DT | ECG | handcrafted | medium | no | Fiducial features, manhattan- and euclidian distances: Qamp, Ramp, Samp, Tamp, QQman, RRman, SSman, TTman, QQeuc, RReuc, SSeuc, TTeuc, Manhattan distances of QR, ST, RS, RT, QS, QT, RST, and QRS for 1/2/3 peaks |
| ECG | 36 | Hexoskin E50 NB | ECG | handcrafted | medium | no | Fiducial features, manhattan- and euclidian distances: Qamp, Ramp, Samp, Tamp, QQman, RRman, SSman, TTman, QQeuc, RReuc, SSeuc, TTeuc, Manhattan distances of QR, ST, RS, RT, QS, QT, RST, and QRS for 1/2/3 peaks |
| ECG | 36 | Hexoskin M30 DT | ECG | handcrafted | medium | no | Fiducial features, manhattan- and euclidian distances: Qamp, Ramp, Samp, Tamp, QQman, RRman, SSman, TTman, QQeuc, RReuc, SSeuc, TTeuc, Manhattan distances of QR, ST, RS, RT, QS, QT, RST, and QRS for 1/2/3 peaks |
| ECG | 36 | Hexoskin M30 NB | ECG | handcrafted | medium | no | Fiducial features, manhattan- and euclidian distances: Qamp, Ramp, Samp, Tamp, QQman, RRman, SSman, TTman, QQeuc, RReuc, SSeuc, TTeuc, Manhattan distances of QR, ST, RS, RT, QS, QT, RST, and QRS for 1/2/3 peaks |
| ECG | 36 | Hexoskin M40 DT | ECG | handcrafted | medium | no | Fiducial features, manhattan- and euclidian distances: Qamp, Ramp, Samp, Tamp, QQman, RRman, SSman, TTman, QQeuc, RReuc, SSeuc, TTeuc, Manhattan distances of QR, ST, RS, RT, QS, QT, RST, and QRS for 1/2/3 peaks |
| ECG | 36 | Hexoskin M40 NB | ECG | handcrafted | medium | no | Fiducial features, manhattan- and euclidian distances: Qamp, Ramp, Samp, Tamp, QQman, RRman, SSman, TTman, QQeuc, RReuc, SSeuc, TTeuc, Manhattan distances of QR, ST, RS, RT, QS, QT, RST, and QRS for 1/2/3 peaks |
| ECG | 36 | Hexoskin M50 DT | ECG | handcrafted | medium | no | Fiducial features, manhattan- and euclidian distances: Qamp, Ramp, Samp, Tamp, QQman, RRman, SSman, TTman, QQeuc, RReuc, SSeuc, TTeuc, Manhattan distances of QR, ST, RS, RT, QS, QT, RST, and QRS for 1/2/3 peaks |
| ECG | 36 | Hexoskin M50 NB | ECG | handcrafted | medium | no | Fiducial features, manhattan- and euclidian distances: Qamp, Ramp, Samp, Tamp, QQman, RRman, SSman, TTman, QQeuc, RReuc, SSeuc, TTeuc, Manhattan distances of QR, ST, RS, RT, QS, QT, RST, and QRS for 1/2/3 peaks |
| ECG | 43 |  | ECG | handcrafted | not coded | no | invariant features of “RR” intervals |
| ECG | 445 | 445.0.1 / A1 | ECG | handcrafted | not coded | no | PQRST morphology of the ECG using the signal of the QRS complex |
| ECG | 445 | 445.0.1 / A2 | ECG | handcrafted | not coded | no | PQRST morphology of the ECG using the signal of the QRS complex |
| ECG | 445 | 445.0.1 / A3 | ECG | handcrafted | not coded | no | PQRST morphology of the ECG using the signal of the QRS complex |
| ECG | 445 | 445.0.1 / B1 | ECG | handcrafted | not coded | no | PQRST morphology of the ECG using the signal of the QRS complex |
| ECG | 445 | 445.0.1 / B2 | ECG | handcrafted | not coded | no | PQRST morphology of the ECG using the signal of the QRS complex |
| ECG | 445 | 445.0.1 / C | ECG | handcrafted | not coded | no | PQRST morphology of the ECG using the signal of the QRS complex |
| ECG | 447 | DensNet201 1s Mel10 | ECG | hybrid | not coded | no | time frequency via mel-spectrogram |
| ECG | 447 | DensNet201 1s Mel2 | ECG | hybrid | not coded | no | time frequency via mel-spectrogram |
| ECG | 447 | DensNet201 1s Mel4 | ECG | hybrid | not coded | no | time frequency via mel-spectrogram |
| ECG | 447 | DensNet201 1s Sc10 | ECG | hybrid | not coded | no | time frequency via scalogram |
| ECG | 447 | DensNet201 1s Sc2 | ECG | hybrid | not coded | no | time frequency via scalogram |
| ECG | 447 | DensNet201 1s Sc4 | ECG | hybrid | not coded | no | time frequency via scalogram |
| ECG | 447 | DensNet201 1s Sp10 | ECG | hybrid | not coded | no | time frequency via spectrogram |
| ECG | 447 | DensNet201 1s Sp2 | ECG | hybrid | not coded | no | time frequency via spectrogram |
| ECG | 447 | DensNet201 1s Sp4 | ECG | hybrid | not coded | no | time frequency via spectrogram |
| ECG | 447 | DensNet201 3s Mel10 | ECG | hybrid | not coded | no | time frequency via mel-spectrogram |
| ECG | 447 | DensNet201 3s Mel2 | ECG | hybrid | not coded | no | time frequency via mel-spectrogram |
| ECG | 447 | DensNet201 3s Mel4 | ECG | hybrid | not coded | no | time frequency via mel-spectrogram |
| ECG | 447 | DensNet201 3s Sc10 | ECG | hybrid | not coded | no | time frequency via scalogram |
| ECG | 447 | DensNet201 3s Sc2 | ECG | hybrid | not coded | no | time frequency via scalogram |
| ECG | 447 | DensNet201 3s Sc4 | ECG | hybrid | not coded | no | time frequency via scalogram |
| ECG | 447 | DensNet201 3s Sp10 | ECG | hybrid | not coded | no | time frequency via spectrogram |
| ECG | 447 | DensNet201 3s Sp2 | ECG | hybrid | not coded | no | time frequency via spectrogram |
| ECG | 447 | DensNet201 3s Sp4 | ECG | hybrid | not coded | no | time frequency via spectrogram |
| ECG | 447 | DensNet201 5s Mel10 | ECG | hybrid | not coded | no | time frequency via mel-spectrogram |
| ECG | 447 | DensNet201 5s Mel2 | ECG | hybrid | not coded | no | time frequency via mel-spectrogram |
| ECG | 447 | DensNet201 5s Mel4 | ECG | hybrid | not coded | no | time frequency via mel-spectrogram |
| ECG | 447 | DensNet201 5s Sc10 | ECG | hybrid | not coded | no | time frequency via scalogram |
| ECG | 447 | DensNet201 5s Sc2 | ECG | hybrid | not coded | no | time frequency via scalogram |
| ECG | 447 | DensNet201 5s Sc4 | ECG | hybrid | not coded | no | time frequency via scalogram |
| ECG | 447 | DensNet201 5s Sp10 | ECG | hybrid | not coded | no | time frequency via spectrogram |
| ECG | 447 | DensNet201 5s Sp2 | ECG | hybrid | not coded | no | time frequency via spectrogram |
| ECG | 447 | DensNet201 5s Sp4 | ECG | hybrid | not coded | no | time frequency via spectrogram |
| ECG | 447 | DenseNet201 1s Mel10 Resting | ECG | hybrid | not coded | no | time frequency via mel-spectrogram |
| ECG | 447 | DenseNet201 1s Mel10 Standing | ECG | hybrid | not coded | no | time frequency via mel-spectrogram |
| ECG | 447 | DenseNet201 1s Mel10 Walking | ECG | hybrid | not coded | no | time frequency via mel-spectrogram |
| ECG | 447 | DenseNet201 1s Mel10 walking | ECG | hybrid | not coded | no | time frequency via mel-spectrogram |
| ECG | 447 | DenseNet201 1s Mel2 Resting | ECG | hybrid | not coded | no | time frequency via mel-spectrogram |
| ECG | 447 | DenseNet201 1s Mel2 Standing | ECG | hybrid | not coded | no | time frequency via mel-spectrogram |
| ECG | 447 | DenseNet201 1s Mel2 Walking | ECG | hybrid | not coded | no | time frequency via mel-spectrogram |
| ECG | 447 | DenseNet201 1s Mel2 walking | ECG | hybrid | not coded | no | time frequency via mel-spectrogram |
| ECG | 447 | DenseNet201 1s Mel4 Resting | ECG | hybrid | not coded | no | time frequency via mel-spectrogram |
| ECG | 447 | DenseNet201 1s Mel4 Standing | ECG | hybrid | not coded | no | time frequency via mel-spectrogram |
| ECG | 447 | DenseNet201 1s Mel4 Walking | ECG | hybrid | not coded | no | time frequency via mel-spectrogram |
| ECG | 447 | DenseNet201 1s Mel4 walking | ECG | hybrid | not coded | no | time frequency via mel-spectrogram |
| ECG | 447 | DenseNet201 1s Sc10 Resting | ECG | hybrid | not coded | no | time frequency via scalogram |
| ECG | 447 | DenseNet201 1s Sc10 Standing | ECG | hybrid | not coded | no | time frequency via scalogram |
| ECG | 447 | DenseNet201 1s Sc10 Walking | ECG | hybrid | not coded | no | time frequency via scalogram |
| ECG | 447 | DenseNet201 1s Sc10 walking | ECG | hybrid | not coded | no | time frequency via scalogram |
| ECG | 447 | DenseNet201 1s Sc2 Resting | ECG | hybrid | not coded | no | time frequency via scalogram |
| ECG | 447 | DenseNet201 1s Sc2 Standing | ECG | hybrid | not coded | no | time frequency via scalogram |
| ECG | 447 | DenseNet201 1s Sc2 Walking | ECG | hybrid | not coded | no | time frequency via scalogram |
| ECG | 447 | DenseNet201 1s Sc2 walking | ECG | hybrid | not coded | no | time frequency via scalogram |
| ECG | 447 | DenseNet201 1s Sc4 Resting | ECG | hybrid | not coded | no | time frequency via scalogram |
| ECG | 447 | DenseNet201 1s Sc4 Standing | ECG | hybrid | not coded | no | time frequency via scalogram |
| ECG | 447 | DenseNet201 1s Sc4 Walking | ECG | hybrid | not coded | no | time frequency via scalogram |
| ECG | 447 | DenseNet201 1s Sc4 walking | ECG | hybrid | not coded | no | time frequency via scalogram |
| ECG | 447 | DenseNet201 1s Sp10 Resting | ECG | hybrid | not coded | no | time frequency via spectrogram |
| ECG | 447 | DenseNet201 1s Sp10 Standing | ECG | hybrid | not coded | no | time frequency via spectrogram |
| ECG | 447 | DenseNet201 1s Sp10 Walking | ECG | hybrid | not coded | no | time frequency via spectrogram |
| ECG | 447 | DenseNet201 1s Sp10 walking | ECG | hybrid | not coded | no | time frequency via spectrogram |
| ECG | 447 | DenseNet201 1s Sp2 Resting | ECG | hybrid | not coded | no | time frequency via spectrogram |
| ECG | 447 | DenseNet201 1s Sp2 Standing | ECG | hybrid | not coded | no | time frequency via spectrogram |
| ECG | 447 | DenseNet201 1s Sp2 Walking | ECG | hybrid | not coded | no | time frequency via spectrogram |
| ECG | 447 | DenseNet201 1s Sp2 walking | ECG | hybrid | not coded | no | time frequency via spectrogram |
| ECG | 447 | DenseNet201 1s Sp4 Resting | ECG | hybrid | not coded | no | time frequency via spectrogram |
| ECG | 447 | DenseNet201 1s Sp4 Standing | ECG | hybrid | not coded | no | time frequency via spectrogram |
| ECG | 447 | DenseNet201 1s Sp4 Walking | ECG | hybrid | not coded | no | time frequency via spectrogram |
| ECG | 447 | DenseNet201 1s Sp4 walking | ECG | hybrid | not coded | no | time frequency via spectrogram |
| ECG | 447 | DenseNet201 3s Mel10 Resting | ECG | hybrid | not coded | no | time frequency via mel-spectrogram |
| ECG | 447 | DenseNet201 3s Mel10 Standing | ECG | hybrid | not coded | no | time frequency via mel-spectrogram |
| ECG | 447 | DenseNet201 3s Mel10 Walking | ECG | hybrid | not coded | no | time frequency via mel-spectrogram |
| ECG | 447 | DenseNet201 3s Mel10 walking | ECG | hybrid | not coded | no | time frequency via mel-spectrogram |
| ECG | 447 | DenseNet201 3s Mel2 Resting | ECG | hybrid | not coded | no | time frequency via mel-spectrogram |
| ECG | 447 | DenseNet201 3s Mel2 Standing | ECG | hybrid | not coded | no | time frequency via mel-spectrogram |
| ECG | 447 | DenseNet201 3s Mel2 Walking | ECG | hybrid | not coded | no | time frequency via mel-spectrogram |
| ECG | 447 | DenseNet201 3s Mel2 walking | ECG | hybrid | not coded | no | time frequency via mel-spectrogram |
| ECG | 447 | DenseNet201 3s Mel4 Resting | ECG | hybrid | not coded | no | time frequency via mel-spectrogram |
| ECG | 447 | DenseNet201 3s Mel4 Standing | ECG | hybrid | not coded | no | time frequency via mel-spectrogram |
| ECG | 447 | DenseNet201 3s Mel4 Walking | ECG | hybrid | not coded | no | time frequency via mel-spectrogram |
| ECG | 447 | DenseNet201 3s Mel4 walking | ECG | hybrid | not coded | no | time frequency via mel-spectrogram |
| ECG | 447 | DenseNet201 3s Sc10 Resting | ECG | hybrid | not coded | no | time frequency via scalogram |
| ECG | 447 | DenseNet201 3s Sc10 Standing | ECG | hybrid | not coded | no | time frequency via scalogram |
| ECG | 447 | DenseNet201 3s Sc10 Walking | ECG | hybrid | not coded | no | time frequency via scalogram |
| ECG | 447 | DenseNet201 3s Sc10 walking | ECG | hybrid | not coded | no | time frequency via scalogram |
| ECG | 447 | DenseNet201 3s Sc2 Resting | ECG | hybrid | not coded | no | time frequency via scalogram |
| ECG | 447 | DenseNet201 3s Sc2 Standing | ECG | hybrid | not coded | no | time frequency via scalogram |
| ECG | 447 | DenseNet201 3s Sc2 Walking | ECG | hybrid | not coded | no | time frequency via scalogram |
| ECG | 447 | DenseNet201 3s Sc2 walking | ECG | hybrid | not coded | no | time frequency via scalogram |
| ECG | 447 | DenseNet201 3s Sc4 Resting | ECG | hybrid | not coded | no | time frequency via scalogram |
| ECG | 447 | DenseNet201 3s Sc4 Standing | ECG | hybrid | not coded | no | time frequency via scalogram |
| ECG | 447 | DenseNet201 3s Sc4 Walking | ECG | hybrid | not coded | no | time frequency via scalogram |
| ECG | 447 | DenseNet201 3s Sc4 walking | ECG | hybrid | not coded | no | time frequency via scalogram |
| ECG | 447 | DenseNet201 3s Sp10 Resting | ECG | hybrid | not coded | no | time frequency via spectrogram |
| ECG | 447 | DenseNet201 3s Sp10 Standing | ECG | hybrid | not coded | no | time frequency via spectrogram |
| ECG | 447 | DenseNet201 3s Sp10 Walking | ECG | hybrid | not coded | no | time frequency via spectrogram |
| ECG | 447 | DenseNet201 3s Sp10 walking | ECG | hybrid | not coded | no | time frequency via spectrogram |
| ECG | 447 | DenseNet201 3s Sp2 Resting | ECG | hybrid | not coded | no | time frequency via spectrogram |
| ECG | 447 | DenseNet201 3s Sp2 Standing | ECG | hybrid | not coded | no | time frequency via spectrogram |
| ECG | 447 | DenseNet201 3s Sp2 Walking | ECG | hybrid | not coded | no | time frequency via spectrogram |
| ECG | 447 | DenseNet201 3s Sp2 walking | ECG | hybrid | not coded | no | time frequency via spectrogram |
| ECG | 447 | DenseNet201 3s Sp4 Resting | ECG | hybrid | not coded | no | time frequency via spectrogram |
| ECG | 447 | DenseNet201 3s Sp4 Standing | ECG | hybrid | not coded | no | time frequency via spectrogram |
| ECG | 447 | DenseNet201 3s Sp4 Walking | ECG | hybrid | not coded | no | time frequency via spectrogram |
| ECG | 447 | DenseNet201 3s Sp4 walking | ECG | hybrid | not coded | no | time frequency via spectrogram |
| ECG | 447 | DenseNet201 5s Mel10 Resting | ECG | hybrid | not coded | no | time frequency via mel-spectrogram |
| ECG | 447 | DenseNet201 5s Mel10 Standing | ECG | hybrid | not coded | no | time frequency via mel-spectrogram |
| ECG | 447 | DenseNet201 5s Mel10 Walking | ECG | hybrid | not coded | no | time frequency via mel-spectrogram |
| ECG | 447 | DenseNet201 5s Mel10 walking | ECG | hybrid | not coded | no | time frequency via mel-spectrogram |
| ECG | 447 | DenseNet201 5s Mel2 Resting | ECG | hybrid | not coded | no | time frequency via mel-spectrogram |
| ECG | 447 | DenseNet201 5s Mel2 Standing | ECG | hybrid | not coded | no | time frequency via mel-spectrogram |
| ECG | 447 | DenseNet201 5s Mel2 Walking | ECG | hybrid | not coded | no | time frequency via mel-spectrogram |
| ECG | 447 | DenseNet201 5s Mel2 walking | ECG | hybrid | not coded | no | time frequency via mel-spectrogram |
| ECG | 447 | DenseNet201 5s Mel4 Resting | ECG | hybrid | not coded | no | time frequency via mel-spectrogram |
| ECG | 447 | DenseNet201 5s Mel4 Standing | ECG | hybrid | not coded | no | time frequency via mel-spectrogram |
| ECG | 447 | DenseNet201 5s Mel4 Walking | ECG | hybrid | not coded | no | time frequency via mel-spectrogram |
| ECG | 447 | DenseNet201 5s Mel4 walking | ECG | hybrid | not coded | no | time frequency via mel-spectrogram |
| ECG | 447 | DenseNet201 5s Sc10 Resting | ECG | hybrid | not coded | no | time frequency via scalogram |
| ECG | 447 | DenseNet201 5s Sc10 Standing | ECG | hybrid | not coded | no | time frequency via scalogram |
| ECG | 447 | DenseNet201 5s Sc10 Walking | ECG | hybrid | not coded | no | time frequency via scalogram |
| ECG | 447 | DenseNet201 5s Sc10 walking | ECG | hybrid | not coded | no | time frequency via scalogram |
| ECG | 447 | DenseNet201 5s Sc2 Resting | ECG | hybrid | not coded | no | time frequency via scalogram |
| ECG | 447 | DenseNet201 5s Sc2 Standing | ECG | hybrid | not coded | no | time frequency via scalogram |
| ECG | 447 | DenseNet201 5s Sc2 Walking | ECG | hybrid | not coded | no | time frequency via scalogram |
| ECG | 447 | DenseNet201 5s Sc2 walking | ECG | hybrid | not coded | no | time frequency via scalogram |
| ECG | 447 | DenseNet201 5s Sc4 Resting | ECG | hybrid | not coded | no | time frequency via scalogram |
| ECG | 447 | DenseNet201 5s Sc4 Standing | ECG | hybrid | not coded | no | time frequency via scalogram |
| ECG | 447 | DenseNet201 5s Sc4 Walking | ECG | hybrid | not coded | no | time frequency via scalogram |
| ECG | 447 | DenseNet201 5s Sc4 walking | ECG | hybrid | not coded | no | time frequency via scalogram |
| ECG | 447 | DenseNet201 5s Sp10 Resting | ECG | hybrid | not coded | no | time frequency via spectrogram |
| ECG | 447 | DenseNet201 5s Sp10 Standing | ECG | hybrid | not coded | no | time frequency via spectrogram |
| ECG | 447 | DenseNet201 5s Sp10 Walking | ECG | hybrid | not coded | no | time frequency via spectrogram |
| ECG | 447 | DenseNet201 5s Sp10 walking | ECG | hybrid | not coded | no | time frequency via spectrogram |
| ECG | 447 | DenseNet201 5s Sp2 Resting | ECG | hybrid | not coded | no | time frequency via spectrogram |
| ECG | 447 | DenseNet201 5s Sp2 Standing | ECG | hybrid | not coded | no | time frequency via spectrogram |
| ECG | 447 | DenseNet201 5s Sp2 Walking | ECG | hybrid | not coded | no | time frequency via spectrogram |
| ECG | 447 | DenseNet201 5s Sp2 walking | ECG | hybrid | not coded | no | time frequency via spectrogram |
| ECG | 447 | DenseNet201 5s Sp4 Resting | ECG | hybrid | not coded | no | time frequency via spectrogram |
| ECG | 447 | DenseNet201 5s Sp4 Standing | ECG | hybrid | not coded | no | time frequency via spectrogram |
| ECG | 447 | DenseNet201 5s Sp4 Walking | ECG | hybrid | not coded | no | time frequency via spectrogram |
| ECG | 447 | DenseNet201 5s Sp4 walking | ECG | hybrid | not coded | no | time frequency via spectrogram |
| ECG | 447 | ResNet50 1s Mel10 | ECG | hybrid | not coded | no | time frequency via mel-spectrogram |
| ECG | 447 | ResNet50 1s Mel10 Resting | ECG | hybrid | not coded | no | time frequency via mel-spectrogram |
| ECG | 447 | ResNet50 1s Mel10 Standing | ECG | hybrid | not coded | no | time frequency via mel-spectrogram |
| ECG | 447 | ResNet50 1s Mel10 Walking | ECG | hybrid | not coded | no | time frequency via mel-spectrogram |
| ECG | 447 | ResNet50 1s Mel10 walking | ECG | hybrid | not coded | no | time frequency via mel-spectrogram |
| ECG | 447 | ResNet50 1s Mel2 | ECG | hybrid | not coded | no | time frequency via mel-spectrogram |
| ECG | 447 | ResNet50 1s Mel2 Resting | ECG | hybrid | not coded | no | time frequency via mel-spectrogram |
| ECG | 447 | ResNet50 1s Mel2 Standing | ECG | hybrid | not coded | no | time frequency via mel-spectrogram |
| ECG | 447 | ResNet50 1s Mel2 Walking | ECG | hybrid | not coded | no | time frequency via mel-spectrogram |
| ECG | 447 | ResNet50 1s Mel2 walking | ECG | hybrid | not coded | no | time frequency via mel-spectrogram |
| ECG | 447 | ResNet50 1s Mel4 | ECG | hybrid | not coded | no | time frequency via mel-spectrogram |
| ECG | 447 | ResNet50 1s Mel4 Resting | ECG | hybrid | not coded | no | time frequency via mel-spectrogram |
| ECG | 447 | ResNet50 1s Mel4 Standing | ECG | hybrid | not coded | no | time frequency via mel-spectrogram |
| ECG | 447 | ResNet50 1s Mel4 Walking | ECG | hybrid | not coded | no | time frequency via mel-spectrogram |
| ECG | 447 | ResNet50 1s Mel4 walking | ECG | hybrid | not coded | no | time frequency via mel-spectrogram |
| ECG | 447 | ResNet50 1s Sc10 | ECG | hybrid | not coded | no | time frequency via scalogram |
| ECG | 447 | ResNet50 1s Sc10 Resting | ECG | hybrid | not coded | no | time frequency via scalogram |
| ECG | 447 | ResNet50 1s Sc10 Standing | ECG | hybrid | not coded | no | time frequency via scalogram |
| ECG | 447 | ResNet50 1s Sc10 Walking | ECG | hybrid | not coded | no | time frequency via scalogram |
| ECG | 447 | ResNet50 1s Sc10 walking | ECG | hybrid | not coded | no | time frequency via scalogram |
| ECG | 447 | ResNet50 1s Sc2 | ECG | hybrid | not coded | no | time frequency via scalogram |
| ECG | 447 | ResNet50 1s Sc2 Resting | ECG | hybrid | not coded | no | time frequency via scalogram |
| ECG | 447 | ResNet50 1s Sc2 Standing | ECG | hybrid | not coded | no | time frequency via scalogram |
| ECG | 447 | ResNet50 1s Sc2 Walking | ECG | hybrid | not coded | no | time frequency via scalogram |
| ECG | 447 | ResNet50 1s Sc2 walking | ECG | hybrid | not coded | no | time frequency via scalogram |
| ECG | 447 | ResNet50 1s Sc4 | ECG | hybrid | not coded | no | time frequency via scalogram |
| ECG | 447 | ResNet50 1s Sc4 Resting | ECG | hybrid | not coded | no | time frequency via scalogram |
| ECG | 447 | ResNet50 1s Sc4 Standing | ECG | hybrid | not coded | no | time frequency via scalogram |
| ECG | 447 | ResNet50 1s Sc4 Walking | ECG | hybrid | not coded | no | time frequency via scalogram |
| ECG | 447 | ResNet50 1s Sc4 walking | ECG | hybrid | not coded | no | time frequency via scalogram |
| ECG | 447 | ResNet50 1s Sp10 | ECG | hybrid | not coded | no | time frequency via spectrogram |
| ECG | 447 | ResNet50 1s Sp10 Resting | ECG | hybrid | not coded | no | time frequency via spectrogram |
| ECG | 447 | ResNet50 1s Sp10 Standing | ECG | hybrid | not coded | no | time frequency via spectrogram |
| ECG | 447 | ResNet50 1s Sp10 Walking | ECG | hybrid | not coded | no | time frequency via spectrogram |
| ECG | 447 | ResNet50 1s Sp10 walking | ECG | hybrid | not coded | no | time frequency via spectrogram |
| ECG | 447 | ResNet50 1s Sp2 | ECG | hybrid | not coded | no | time frequency via spectrogram |
| ECG | 447 | ResNet50 1s Sp2 Resting | ECG | hybrid | not coded | no | time frequency via spectrogram |
| ECG | 447 | ResNet50 1s Sp2 Standing | ECG | hybrid | not coded | no | time frequency via spectrogram |
| ECG | 447 | ResNet50 1s Sp2 Walking | ECG | hybrid | not coded | no | time frequency via spectrogram |
| ECG | 447 | ResNet50 1s Sp2 walking | ECG | hybrid | not coded | no | time frequency via spectrogram |
| ECG | 447 | ResNet50 1s Sp4 | ECG | hybrid | not coded | no | time frequency via spectrogram |
| ECG | 447 | ResNet50 1s Sp4 Resting | ECG | hybrid | not coded | no | time frequency via spectrogram |
| ECG | 447 | ResNet50 1s Sp4 Standing | ECG | hybrid | not coded | no | time frequency via spectrogram |
| ECG | 447 | ResNet50 1s Sp4 Walking | ECG | hybrid | not coded | no | time frequency via spectrogram |
| ECG | 447 | ResNet50 1s Sp4 walking | ECG | hybrid | not coded | no | time frequency via spectrogram |
| ECG | 447 | ResNet50 3s Mel10 | ECG | hybrid | not coded | no | time frequency via mel-spectrogram |
| ECG | 447 | ResNet50 3s Mel10 Resting | ECG | hybrid | not coded | no | time frequency via mel-spectrogram |
| ECG | 447 | ResNet50 3s Mel10 Standing | ECG | hybrid | not coded | no | time frequency via mel-spectrogram |
| ECG | 447 | ResNet50 3s Mel10 Walking | ECG | hybrid | not coded | no | time frequency via mel-spectrogram |
| ECG | 447 | ResNet50 3s Mel10 walking | ECG | hybrid | not coded | no | time frequency via mel-spectrogram |
| ECG | 447 | ResNet50 3s Mel2 | ECG | hybrid | not coded | no | time frequency via mel-spectrogram |
| ECG | 447 | ResNet50 3s Mel2 Resting | ECG | hybrid | not coded | no | time frequency via mel-spectrogram |
| ECG | 447 | ResNet50 3s Mel2 Standing | ECG | hybrid | not coded | no | time frequency via mel-spectrogram |
| ECG | 447 | ResNet50 3s Mel2 Walking | ECG | hybrid | not coded | no | time frequency via mel-spectrogram |
| ECG | 447 | ResNet50 3s Mel2 walking | ECG | hybrid | not coded | no | time frequency via mel-spectrogram |
| ECG | 447 | ResNet50 3s Mel4 | ECG | hybrid | not coded | no | time frequency via mel-spectrogram |
| ECG | 447 | ResNet50 3s Mel4 Resting | ECG | hybrid | not coded | no | time frequency via mel-spectrogram |
| ECG | 447 | ResNet50 3s Mel4 Standing | ECG | hybrid | not coded | no | time frequency via mel-spectrogram |
| ECG | 447 | ResNet50 3s Mel4 Walking | ECG | hybrid | not coded | no | time frequency via mel-spectrogram |
| ECG | 447 | ResNet50 3s Mel4 walking | ECG | hybrid | not coded | no | time frequency via mel-spectrogram |
| ECG | 447 | ResNet50 3s Sc10 | ECG | hybrid | not coded | no | time frequency via scalogram |
| ECG | 447 | ResNet50 3s Sc10 Resting | ECG | hybrid | not coded | no | time frequency via scalogram |
| ECG | 447 | ResNet50 3s Sc10 Standing | ECG | hybrid | not coded | no | time frequency via scalogram |
| ECG | 447 | ResNet50 3s Sc10 Walking | ECG | hybrid | not coded | no | time frequency via scalogram |
| ECG | 447 | ResNet50 3s Sc10 walking | ECG | hybrid | not coded | no | time frequency via scalogram |
| ECG | 447 | ResNet50 3s Sc2 | ECG | hybrid | not coded | no | time frequency via scalogram |
| ECG | 447 | ResNet50 3s Sc2 Resting | ECG | hybrid | not coded | no | time frequency via scalogram |
| ECG | 447 | ResNet50 3s Sc2 Standing | ECG | hybrid | not coded | no | time frequency via scalogram |
| ECG | 447 | ResNet50 3s Sc2 Walking | ECG | hybrid | not coded | no | time frequency via scalogram |
| ECG | 447 | ResNet50 3s Sc2 walking | ECG | hybrid | not coded | no | time frequency via scalogram |
| ECG | 447 | ResNet50 3s Sc4 | ECG | hybrid | not coded | no | time frequency via scalogram |
| ECG | 447 | ResNet50 3s Sc4 Resting | ECG | hybrid | not coded | no | time frequency via scalogram |
| ECG | 447 | ResNet50 3s Sc4 Standing | ECG | hybrid | not coded | no | time frequency via scalogram |
| ECG | 447 | ResNet50 3s Sc4 Walking | ECG | hybrid | not coded | no | time frequency via scalogram |
| ECG | 447 | ResNet50 3s Sc4 walking | ECG | hybrid | not coded | no | time frequency via scalogram |
| ECG | 447 | ResNet50 3s Sp10 | ECG | hybrid | not coded | no | time frequency via spectrogram |
| ECG | 447 | ResNet50 3s Sp10 Resting | ECG | hybrid | not coded | no | time frequency via spectrogram |
| ECG | 447 | ResNet50 3s Sp10 Standing | ECG | hybrid | not coded | no | time frequency via spectrogram |
| ECG | 447 | ResNet50 3s Sp10 Walking | ECG | hybrid | not coded | no | time frequency via spectrogram |
| ECG | 447 | ResNet50 3s Sp10 walking | ECG | hybrid | not coded | no | time frequency via spectrogram |
| ECG | 447 | ResNet50 3s Sp2 | ECG | hybrid | not coded | no | time frequency via spectrogram |
| ECG | 447 | ResNet50 3s Sp2 Resting | ECG | hybrid | not coded | no | time frequency via spectrogram |
| ECG | 447 | ResNet50 3s Sp2 Standing | ECG | hybrid | not coded | no | time frequency via spectrogram |
| ECG | 447 | ResNet50 3s Sp2 Walking | ECG | hybrid | not coded | no | time frequency via spectrogram |
| ECG | 447 | ResNet50 3s Sp2 walking | ECG | hybrid | not coded | no | time frequency via spectrogram |
| ECG | 447 | ResNet50 3s Sp4 | ECG | hybrid | not coded | no | time frequency via spectrogram |
| ECG | 447 | ResNet50 3s Sp4 Resting | ECG | hybrid | not coded | no | time frequency via spectrogram |
| ECG | 447 | ResNet50 3s Sp4 Standing | ECG | hybrid | not coded | no | time frequency via spectrogram |
| ECG | 447 | ResNet50 3s Sp4 Walking | ECG | hybrid | not coded | no | time frequency via spectrogram |
| ECG | 447 | ResNet50 3s Sp4 walking | ECG | hybrid | not coded | no | time frequency via spectrogram |
| ECG | 447 | ResNet50 5s Mel10 | ECG | hybrid | not coded | no | time frequency via mel-spectrogram |
| ECG | 447 | ResNet50 5s Mel10 Resting | ECG | hybrid | not coded | no | time frequency via mel-spectrogram |
| ECG | 447 | ResNet50 5s Mel10 Standing | ECG | hybrid | not coded | no | time frequency via mel-spectrogram |
| ECG | 447 | ResNet50 5s Mel10 Walking | ECG | hybrid | not coded | no | time frequency via mel-spectrogram |
| ECG | 447 | ResNet50 5s Mel10 walking | ECG | hybrid | not coded | no | time frequency via mel-spectrogram |
| ECG | 447 | ResNet50 5s Mel2 | ECG | hybrid | not coded | no | time frequency via mel-spectrogram |
| ECG | 447 | ResNet50 5s Mel2 Resting | ECG | hybrid | not coded | no | time frequency via mel-spectrogram |
| ECG | 447 | ResNet50 5s Mel2 Standing | ECG | hybrid | not coded | no | time frequency via mel-spectrogram |
| ECG | 447 | ResNet50 5s Mel2 Walking | ECG | hybrid | not coded | no | time frequency via mel-spectrogram |
| ECG | 447 | ResNet50 5s Mel2 walking | ECG | hybrid | not coded | no | time frequency via mel-spectrogram |
| ECG | 447 | ResNet50 5s Mel4 | ECG | hybrid | not coded | no | time frequency via mel-spectrogram |
| ECG | 447 | ResNet50 5s Mel4 Resting | ECG | hybrid | not coded | no | time frequency via mel-spectrogram |
| ECG | 447 | ResNet50 5s Mel4 Standing | ECG | hybrid | not coded | no | time frequency via mel-spectrogram |
| ECG | 447 | ResNet50 5s Mel4 Walking | ECG | hybrid | not coded | no | time frequency via mel-spectrogram |
| ECG | 447 | ResNet50 5s Mel4 walking | ECG | hybrid | not coded | no | time frequency via mel-spectrogram |
| ECG | 447 | ResNet50 5s Sc10 | ECG | hybrid | not coded | no | time frequency via scalogram |
| ECG | 447 | ResNet50 5s Sc10 Resting | ECG | hybrid | not coded | no | time frequency via scalogram |
| ECG | 447 | ResNet50 5s Sc10 Standing | ECG | hybrid | not coded | no | time frequency via scalogram |
| ECG | 447 | ResNet50 5s Sc10 Walking | ECG | hybrid | not coded | no | time frequency via scalogram |
| ECG | 447 | ResNet50 5s Sc10 walking | ECG | hybrid | not coded | no | time frequency via scalogram |
| ECG | 447 | ResNet50 5s Sc2 | ECG | hybrid | not coded | no | time frequency via scalogram |
| ECG | 447 | ResNet50 5s Sc2 Resting | ECG | hybrid | not coded | no | time frequency via scalogram |
| ECG | 447 | ResNet50 5s Sc2 Standing | ECG | hybrid | not coded | no | time frequency via scalogram |
| ECG | 447 | ResNet50 5s Sc2 Walking | ECG | hybrid | not coded | no | time frequency via scalogram |
| ECG | 447 | ResNet50 5s Sc2 walking | ECG | hybrid | not coded | no | time frequency via scalogram |
| ECG | 447 | ResNet50 5s Sc4 | ECG | hybrid | not coded | no | time frequency via scalogram |
| ECG | 447 | ResNet50 5s Sc4 Resting | ECG | hybrid | not coded | no | time frequency via scalogram |
| ECG | 447 | ResNet50 5s Sc4 Standing | ECG | hybrid | not coded | no | time frequency via scalogram |
| ECG | 447 | ResNet50 5s Sc4 Walking | ECG | hybrid | not coded | no | time frequency via scalogram |
| ECG | 447 | ResNet50 5s Sc4 walking | ECG | hybrid | not coded | no | time frequency via scalogram |
| ECG | 447 | ResNet50 5s Sp10 | ECG | hybrid | not coded | no | time frequency via spectrogram |
| ECG | 447 | ResNet50 5s Sp10 Resting | ECG | hybrid | not coded | no | time frequency via spectrogram |
| ECG | 447 | ResNet50 5s Sp10 Standing | ECG | hybrid | not coded | no | time frequency via spectrogram |
| ECG | 447 | ResNet50 5s Sp10 Walking | ECG | hybrid | not coded | no | time frequency via spectrogram |
| ECG | 447 | ResNet50 5s Sp10 walking | ECG | hybrid | not coded | no | time frequency via spectrogram |
| ECG | 447 | ResNet50 5s Sp2 | ECG | hybrid | not coded | no | time frequency via spectrogram |
| ECG | 447 | ResNet50 5s Sp2 Resting | ECG | hybrid | not coded | no | time frequency via spectrogram |
| ECG | 447 | ResNet50 5s Sp2 Standing | ECG | hybrid | not coded | no | time frequency via spectrogram |
| ECG | 447 | ResNet50 5s Sp2 Walking | ECG | hybrid | not coded | no | time frequency via spectrogram |
| ECG | 447 | ResNet50 5s Sp2 walking | ECG | hybrid | not coded | no | time frequency via spectrogram |
| ECG | 447 | ResNet50 5s Sp4 | ECG | hybrid | not coded | no | time frequency via spectrogram |
| ECG | 447 | ResNet50 5s Sp4 Resting | ECG | hybrid | not coded | no | time frequency via spectrogram |
| ECG | 447 | ResNet50 5s Sp4 Standing | ECG | hybrid | not coded | no | time frequency via spectrogram |
| ECG | 447 | ResNet50 5s Sp4 Walking | ECG | hybrid | not coded | no | time frequency via spectrogram |
| ECG | 447 | ResNet50 5s Sp4 walking | ECG | hybrid | not coded | no | time frequency via spectrogram |
| ECG | 455 | DL | ECG | deep | not coded | no | time / frequency domain features implicit via DL |
| ECG | 455 | time | ECG | hybrid | medium | no | time-domain features – intervals and amplitudes from each heartbeat and Hjorth descriptors of an average heartbeat. 14 interval features, 2 amplitude features, 5 Hjorth features, and one feature related to interval variability |
| ECG | 455 | wavelet | ECG | hybrid | high | no | Daubechies wavelets (db3, db4, db5) with three, four and five levels of decomposition. |
| ECG | 460 | 460.0.1 | ECG | hybrid | not coded | yes | Higher-order statistical features (cumulants), implicitly extracted |
| ECG | 475 | AE / PM 1 subject | ECG | deep | not coded | no | waveform of single ECG signals in sequence |
| ECG | 475 | AE / PM 2 subject | ECG | deep | not coded | no | waveform of single ECG signals in sequence |
| ECG | 475 | AE / PM 3 subject | ECG | deep | not coded | no | waveform of single ECG signals in sequence |
| ECG | 475 | AE / PM 4 subject | ECG | deep | not coded | no | waveform of single ECG signals in sequence |
| ECG | 475 | AE / PM 5 subject | ECG | deep | not coded | no | waveform of single ECG signals in sequence |
| ECG | 475 | LSTM / PM 1 subject | ECG | deep | not coded | no | waveform of single ECG signals in sequence |
| ECG | 475 | LSTM / PM 2 subject | ECG | deep | not coded | no | waveform of single ECG signals in sequence |
| ECG | 475 | LSTM / PM 3 subject | ECG | deep | not coded | no | waveform of single ECG signals in sequence |
| ECG | 475 | LSTM / PM 4 subject | ECG | deep | not coded | no | waveform of single ECG signals in sequence |
| ECG | 475 | LSTM / PM 5 subject | ECG | deep | not coded | no | waveform of single ECG signals in sequence |
| ECG | 495 | S1 | ECG | handcrafted | not coded | no | Extracted from PQRST complex |
| ECG | 495 | S2 | ECG | handcrafted | not coded | no | Extracted from PQRST complex |
| ECG | 495 | S3 | ECG | handcrafted | not coded | no | Extracted from PQRST complex |
| ECG | 495 | S4 | ECG | handcrafted | not coded | no | Extracted from PQRST complex |
| ECG | 495 | S5 | ECG | handcrafted | not coded | no | Extracted from PQRST complex |
| PPG | 205 | PPG | PPG | hybrid | not coded | no | Instantaneous Frequency (IF) and spectral entropy (SE) |
| PPG | 266 | 15 15 20 15 | PPG | deep | not coded | no | time series data (Automatic feature extraction via DL) |
| PPG | 266 | 15 15 30 15 | PPG | deep | not coded | no | time series data (Automatic feature extraction via DL) |
| PPG | 266 | 15 15 40 15 | PPG | deep | not coded | no | time series data (Automatic feature extraction via DL) |
| PPG | 266 | 15 15 50 15 | PPG | deep | not coded | no | time series data (Automatic feature extraction via DL) |
| PPG | 266 | 30 15 20 15 | PPG | deep | not coded | no | time series data (Automatic feature extraction via DL) |
| PPG | 266 | 30 15 30 15 | PPG | deep | not coded | no | time series data (Automatic feature extraction via DL) |
| PPG | 266 | 30 15 40 15 | PPG | deep | not coded | no | time series data (Automatic feature extraction via DL) |
| PPG | 266 | 30 15 50 15 | PPG | deep | not coded | no | time series data (Automatic feature extraction via DL) |
| PPG | 266 | 30 32 20 15 | PPG | deep | not coded | no | time series data (Automatic feature extraction via DL) |
| PPG | 266 | 30 32 30 32 | PPG | deep | not coded | no | time series data (Automatic feature extraction via DL) |
| PPG | 266 | 30 32 40 32 | PPG | deep | not coded | no | time series data (Automatic feature extraction via DL) |
| PPG | 266 | 30 32 50 32 | PPG | deep | not coded | no | time series data (Automatic feature extraction via DL) |
| PPG | 278 | S1 idle - S1 touchpad PPG | PPG | handcrafted | not coded | no | partially fiducial approach using the waveform amplitude values |
| PPG | 278 | S1 idle - S1 typing 2 PPG | PPG | handcrafted | not coded | no | partially fiducial approach using the waveform amplitude values |
| PPG | 278 | S1 idle - S1 typing1 PPG | PPG | handcrafted | not coded | no | partially fiducial approach using the waveform amplitude values |
| PPG | 278 | S1 idle - S2 idle PPG | PPG | handcrafted | not coded | no | partially fiducial approach using the waveform amplitude values |
| PPG | 28 |  | PPG | deep | not coded | no | time series data (Automatic feature extraction via DL) |
| PPG | 298 | CEEMDAN + FastICA 12s | PPG | handcrafted | low | no | five fiducial features that only require a single systolic peak to extract the unique cardiac features from the PPG segment and its second derivative (systolic amplitude, pulse width, pulse width / systolic amplitude, crest time, ratio of amplitude of b-wave and a-wave) |
| PPG | 298 | CEEMDAN + FastICA 24s | PPG | handcrafted | low | no | five fiducial features that only require a single systolic peak to extract the unique cardiac features from the PPG segment and its second derivative (systolic amplitude, pulse width, pulse width / systolic amplitude, crest time, ratio of amplitude of b-wave and a-wave) |
| PPG | 298 | VMD+FastICA 12s | PPG | handcrafted | low | no | five fiducial features that only require a single systolic peak to extract the unique cardiac features from the PPG segment and its second derivative (systolic amplitude, pulse width, pulse width / systolic amplitude, crest time, ratio of amplitude of b-wave and a-wave) |
| PPG | 298 | VMD+FastICA 24s | PPG | handcrafted | low | no | five fiducial features that only require a single systolic peak to extract the unique cardiac features from the PPG segment and its second derivative (systolic amplitude, pulse width, pulse width / systolic amplitude, crest time, ratio of amplitude of b-wave and a-wave) |
| PPG | 298 | far-wrist no MA removal | PPG | handcrafted | low | no | five fiducial features that only require a single systolic peak to extract the unique cardiac features from the PPG segment and its second derivative (systolic amplitude, pulse width, pulse width / systolic amplitude, crest time, ratio of amplitude of b-wave and a-wave) |
| PPG | 298 | far-wrist MA removal | PPG | handcrafted | low | no | five fiducial features that only require a single systolic peak to extract the unique cardiac features from the PPG segment and its second derivative (systolic amplitude, pulse width, pulse width / systolic amplitude, crest time, ratio of amplitude of b-wave and a-wave) |
| PPG | 298 | near-wrist no MA removal | PPG | handcrafted | low | no | five fiducial features that only require a single systolic peak to extract the unique cardiac features from the PPG segment and its second derivative (systolic amplitude, pulse width, pulse width / systolic amplitude, crest time, ratio of amplitude of b-wave and a-wave) |
| PPG | 298 | near-wrist MA removal | PPG | handcrafted | low | no | five fiducial features that only require a single systolic peak to extract the unique cardiac features from the PPG segment and its second derivative (systolic amplitude, pulse width, pulse width / systolic amplitude, crest time, ratio of amplitude of b-wave and a-wave) |
| PPG | 298 | static 1min | PPG | handcrafted | low | no | five fiducial features that only require a single systolic peak to extract the unique cardiac features from the PPG segment and its second derivative (systolic amplitude, pulse width, pulse width / systolic amplitude, crest time, ratio of amplitude of b-wave and a-wave) |
| PPG | 298 | static 25hz | PPG | handcrafted | low | no | five fiducial features that only require a single systolic peak to extract the unique cardiac features from the PPG segment and its second derivative (systolic amplitude, pulse width, pulse width / systolic amplitude, crest time, ratio of amplitude of b-wave and a-wave) |
| PPG | 298 | static 2h | PPG | handcrafted | low | no | five fiducial features that only require a single systolic peak to extract the unique cardiac features from the PPG segment and its second derivative (systolic amplitude, pulse width, pulse width / systolic amplitude, crest time, ratio of amplitude of b-wave and a-wave) |
| PPG | 298 | static 3h | PPG | handcrafted | low | no | five fiducial features that only require a single systolic peak to extract the unique cardiac features from the PPG segment and its second derivative (systolic amplitude, pulse width, pulse width / systolic amplitude, crest time, ratio of amplitude of b-wave and a-wave) |
| PPG | 298 | static 4 windows | PPG | handcrafted | low | no | five fiducial features that only require a single systolic peak to extract the unique cardiac features from the PPG segment and its second derivative (systolic amplitude, pulse width, pulse width / systolic amplitude, crest time, ratio of amplitude of b-wave and a-wave) |
| PPG | 298 | static 5min | PPG | handcrafted | low | no | five fiducial features that only require a single systolic peak to extract the unique cardiac features from the PPG segment and its second derivative (systolic amplitude, pulse width, pulse width / systolic amplitude, crest time, ratio of amplitude of b-wave and a-wave) |
| PPG | 298 | static 6 windows | PPG | handcrafted | low | no | five fiducial features that only require a single systolic peak to extract the unique cardiac features from the PPG segment and its second derivative (systolic amplitude, pulse width, pulse width / systolic amplitude, crest time, ratio of amplitude of b-wave and a-wave) |
| PPG | 298 | static GBT | PPG | handcrafted | low | no | five fiducial features that only require a single systolic peak to extract the unique cardiac features from the PPG segment and its second derivative (systolic amplitude, pulse width, pulse width / systolic amplitude, crest time, ratio of amplitude of b-wave and a-wave) |
| PPG | 298 | static NN | PPG | handcrafted | low | no | five fiducial features that only require a single systolic peak to extract the unique cardiac features from the PPG segment and its second derivative (systolic amplitude, pulse width, pulse width / systolic amplitude, crest time, ratio of amplitude of b-wave and a-wave) |
| PPG | 298 | static SVM | PPG | handcrafted | low | no | five fiducial features that only require a single systolic peak to extract the unique cardiac features from the PPG segment and its second derivative (systolic amplitude, pulse width, pulse width / systolic amplitude, crest time, ratio of amplitude of b-wave and a-wave) |
| PPG | 298 | static adaptive 3h | PPG | handcrafted | low | no | five fiducial features that only require a single systolic peak to extract the unique cardiac features from the PPG segment and its second derivative (systolic amplitude, pulse width, pulse width / systolic amplitude, crest time, ratio of amplitude of b-wave and a-wave) |
| PPG | 305 | LR 120 EDA-HRV | EDA, PPG | handcrafted | low | no | EDA features: Mean Tonic Component, SD Tonic (Standard deviation of the tonic component), Perc20 (20th percentile of the phasic component), Perc80 Tonic (80th percentile of the tonic component), Quartdev Tonic (Quartile deviation 75 percentile - 25 percentile of the tonic component), Strong Peaks (The number of strong peaks per 100 seconds) and Peaks Phasic (The number of peaks per 100 seconds). HRV features: Time Domain Features: Mean HR, standard deviation of the inter-beat interval (STD RR), mean value of the inter-beat (RR) intervals (Mean RR), root mean square of successive difference of the RR intervals (RMSSD), the percentage of the number of successive RR intervals varying more than 50ms from the previous interval (pNN50), the total number of RR intervals divided by the height of the histogram of all RR intervals measured on a scale with bins of 1/128 s (HRV triangular index), and triangular interpolation of RR interval histogram (TINN). Frequency Domain Features: Low frequency power (LF), high-frequency power (HF), very low frequency power (VLF), prevalent low frequency (pLF), prevalent high frequency (pHF), the ratio of LF to HF (LF/HF) (Preprocessing +FFT), LF, HF, LF/HF (Lomb-Scargle) |
| PPG | 305 | LR 120 HRV | PPG | handcrafted | low | no | Time Domain Features: Mean HR, standard deviation of the inter-beat interval (STD RR), mean value of the inter-beat (RR) intervals (Mean RR), root mean square of successive difference of the RR intervals (RMSSD), the percentage of the number of successive RR intervals varying more than 50ms from the previous interval (pNN50), the total number of RR intervals divided by the height of the histogram of all RR intervals measured on a scale with bins of 1/128 s (HRV triangular index), and triangular interpolation of RR interval histogram (TINN). Frequency Domain Features: Low frequency power (LF), high-frequency power (HF), very low frequency power (VLF), prevalent low frequency (pLF), prevalent high frequency (pHF), the ratio of LF to HF (LF/HF) (Preprocessing +FFT), LF, HF, LF/HF (Lomb-Scargle) |
| PPG | 305 | LR 30 EDA-HRV | EDA, PPG | handcrafted | low | no | EDA features: Mean Tonic Component, SD Tonic (Standard deviation of the tonic component), Perc20 (20th percentile of the phasic component), Perc80 Tonic (80th percentile of the tonic component), Quartdev Tonic (Quartile deviation 75 percentile - 25 percentile of the tonic component), Strong Peaks (The number of strong peaks per 100 seconds) and Peaks Phasic (The number of peaks per 100 seconds). HRV features: Time Domain Features: Mean HR, standard deviation of the inter-beat interval (STD RR), mean value of the inter-beat (RR) intervals (Mean RR), root mean square of successive difference of the RR intervals (RMSSD), the percentage of the number of successive RR intervals varying more than 50ms from the previous interval (pNN50), the total number of RR intervals divided by the height of the histogram of all RR intervals measured on a scale with bins of 1/128 s (HRV triangular index), and triangular interpolation of RR interval histogram (TINN). Frequency Domain Features: Low frequency power (LF), high-frequency power (HF), very low frequency power (VLF), prevalent low frequency (pLF), prevalent high frequency (pHF), the ratio of LF to HF (LF/HF) (Preprocessing +FFT), LF, HF, LF/HF (Lomb-Scargle) |
| PPG | 305 | LR 30 HRV | PPG | handcrafted | low | no | Time Domain Features: Mean HR, standard deviation of the inter-beat interval (STD RR), mean value of the inter-beat (RR) intervals (Mean RR), root mean square of successive difference of the RR intervals (RMSSD), the percentage of the number of successive RR intervals varying more than 50ms from the previous interval (pNN50), the total number of RR intervals divided by the height of the histogram of all RR intervals measured on a scale with bins of 1/128 s (HRV triangular index), and triangular interpolation of RR interval histogram (TINN). Frequency Domain Features: Low frequency power (LF), high-frequency power (HF), very low frequency power (VLF), prevalent low frequency (pLF), prevalent high frequency (pHF), the ratio of LF to HF (LF/HF) (Preprocessing +FFT), LF, HF, LF/HF (Lomb-Scargle) |
| PPG | 305 | LR 60 EDA-HRV | EDA, PPG | handcrafted | low | no | EDA features: Mean Tonic Component, SD Tonic (Standard deviation of the tonic component), Perc20 (20th percentile of the phasic component), Perc80 Tonic (80th percentile of the tonic component), Quartdev Tonic (Quartile deviation 75 percentile - 25 percentile of the tonic component), Strong Peaks (The number of strong peaks per 100 seconds) and Peaks Phasic (The number of peaks per 100 seconds). HRV features: Time Domain Features: Mean HR, standard deviation of the inter-beat interval (STD RR), mean value of the inter-beat (RR) intervals (Mean RR), root mean square of successive difference of the RR intervals (RMSSD), the percentage of the number of successive RR intervals varying more than 50ms from the previous interval (pNN50), the total number of RR intervals divided by the height of the histogram of all RR intervals measured on a scale with bins of 1/128 s (HRV triangular index), and triangular interpolation of RR interval histogram (TINN). Frequency Domain Features: Low frequency power (LF), high-frequency power (HF), very low frequency power (VLF), prevalent low frequency (pLF), prevalent high frequency (pHF), the ratio of LF to HF (LF/HF) (Preprocessing +FFT), LF, HF, LF/HF (Lomb-Scargle) |
| PPG | 305 | LR 60 HRV | PPG | handcrafted | low | no | Time Domain Features: Mean HR, standard deviation of the inter-beat interval (STD RR), mean value of the inter-beat (RR) intervals (Mean RR), root mean square of successive difference of the RR intervals (RMSSD), the percentage of the number of successive RR intervals varying more than 50ms from the previous interval (pNN50), the total number of RR intervals divided by the height of the histogram of all RR intervals measured on a scale with bins of 1/128 s (HRV triangular index), and triangular interpolation of RR interval histogram (TINN). Frequency Domain Features: Low frequency power (LF), high-frequency power (HF), very low frequency power (VLF), prevalent low frequency (pLF), prevalent high frequency (pHF), the ratio of LF to HF (LF/HF) (Preprocessing +FFT), LF, HF, LF/HF (Lomb-Scargle) |
| PPG | 305 | MLP 120 EDA-HRV | EDA, PPG | handcrafted | low | no | EDA features: Mean Tonic Component, SD Tonic (Standard deviation of the tonic component), Perc20 (20th percentile of the phasic component), Perc80 Tonic (80th percentile of the tonic component), Quartdev Tonic (Quartile deviation 75 percentile - 25 percentile of the tonic component), Strong Peaks (The number of strong peaks per 100 seconds) and Peaks Phasic (The number of peaks per 100 seconds). HRV features: Time Domain Features: Mean HR, standard deviation of the inter-beat interval (STD RR), mean value of the inter-beat (RR) intervals (Mean RR), root mean square of successive difference of the RR intervals (RMSSD), the percentage of the number of successive RR intervals varying more than 50ms from the previous interval (pNN50), the total number of RR intervals divided by the height of the histogram of all RR intervals measured on a scale with bins of 1/128 s (HRV triangular index), and triangular interpolation of RR interval histogram (TINN). Frequency Domain Features: Low frequency power (LF), high-frequency power (HF), very low frequency power (VLF), prevalent low frequency (pLF), prevalent high frequency (pHF), the ratio of LF to HF (LF/HF) (Preprocessing +FFT), LF, HF, LF/HF (Lomb-Scargle) |
| PPG | 305 | MLP 120 HRV | PPG | handcrafted | low | no | Time Domain Features: Mean HR, standard deviation of the inter-beat interval (STD RR), mean value of the inter-beat (RR) intervals (Mean RR), root mean square of successive difference of the RR intervals (RMSSD), the percentage of the number of successive RR intervals varying more than 50ms from the previous interval (pNN50), the total number of RR intervals divided by the height of the histogram of all RR intervals measured on a scale with bins of 1/128 s (HRV triangular index), and triangular interpolation of RR interval histogram (TINN). Frequency Domain Features: Low frequency power (LF), high-frequency power (HF), very low frequency power (VLF), prevalent low frequency (pLF), prevalent high frequency (pHF), the ratio of LF to HF (LF/HF) (Preprocessing +FFT), LF, HF, LF/HF (Lomb-Scargle) |
| PPG | 305 | MLP 30 EDA-HRV | EDA, PPG | handcrafted | low | no | EDA features: Mean Tonic Component, SD Tonic (Standard deviation of the tonic component), Perc20 (20th percentile of the phasic component), Perc80 Tonic (80th percentile of the tonic component), Quartdev Tonic (Quartile deviation 75 percentile - 25 percentile of the tonic component), Strong Peaks (The number of strong peaks per 100 seconds) and Peaks Phasic (The number of peaks per 100 seconds). HRV features: Time Domain Features: Mean HR, standard deviation of the inter-beat interval (STD RR), mean value of the inter-beat (RR) intervals (Mean RR), root mean square of successive difference of the RR intervals (RMSSD), the percentage of the number of successive RR intervals varying more than 50ms from the previous interval (pNN50), the total number of RR intervals divided by the height of the histogram of all RR intervals measured on a scale with bins of 1/128 s (HRV triangular index), and triangular interpolation of RR interval histogram (TINN). Frequency Domain Features: Low frequency power (LF), high-frequency power (HF), very low frequency power (VLF), prevalent low frequency (pLF), prevalent high frequency (pHF), the ratio of LF to HF (LF/HF) (Preprocessing +FFT), LF, HF, LF/HF (Lomb-Scargle) |
| PPG | 305 | MLP 30 HRV | PPG | handcrafted | low | no | Time Domain Features: Mean HR, standard deviation of the inter-beat interval (STD RR), mean value of the inter-beat (RR) intervals (Mean RR), root mean square of successive difference of the RR intervals (RMSSD), the percentage of the number of successive RR intervals varying more than 50ms from the previous interval (pNN50), the total number of RR intervals divided by the height of the histogram of all RR intervals measured on a scale with bins of 1/128 s (HRV triangular index), and triangular interpolation of RR interval histogram (TINN). Frequency Domain Features: Low frequency power (LF), high-frequency power (HF), very low frequency power (VLF), prevalent low frequency (pLF), prevalent high frequency (pHF), the ratio of LF to HF (LF/HF) (Preprocessing +FFT), LF, HF, LF/HF (Lomb-Scargle) |
| PPG | 305 | MLP 60 EDA-HRV | EDA, PPG | handcrafted | low | no | EDA features: Mean Tonic Component, SD Tonic (Standard deviation of the tonic component), Perc20 (20th percentile of the phasic component), Perc80 Tonic (80th percentile of the tonic component), Quartdev Tonic (Quartile deviation 75 percentile - 25 percentile of the tonic component), Strong Peaks (The number of strong peaks per 100 seconds) and Peaks Phasic (The number of peaks per 100 seconds). HRV features: Time Domain Features: Mean HR, standard deviation of the inter-beat interval (STD RR), mean value of the inter-beat (RR) intervals (Mean RR), root mean square of successive difference of the RR intervals (RMSSD), the percentage of the number of successive RR intervals varying more than 50ms from the previous interval (pNN50), the total number of RR intervals divided by the height of the histogram of all RR intervals measured on a scale with bins of 1/128 s (HRV triangular index), and triangular interpolation of RR interval histogram (TINN). Frequency Domain Features: Low frequency power (LF), high-frequency power (HF), very low frequency power (VLF), prevalent low frequency (pLF), prevalent high frequency (pHF), the ratio of LF to HF (LF/HF) (Preprocessing +FFT), LF, HF, LF/HF (Lomb-Scargle) |
| PPG | 305 | MLP 60 HRV | PPG | handcrafted | low | no | Time Domain Features: Mean HR, standard deviation of the inter-beat interval (STD RR), mean value of the inter-beat (RR) intervals (Mean RR), root mean square of successive difference of the RR intervals (RMSSD), the percentage of the number of successive RR intervals varying more than 50ms from the previous interval (pNN50), the total number of RR intervals divided by the height of the histogram of all RR intervals measured on a scale with bins of 1/128 s (HRV triangular index), and triangular interpolation of RR interval histogram (TINN). Frequency Domain Features: Low frequency power (LF), high-frequency power (HF), very low frequency power (VLF), prevalent low frequency (pLF), prevalent high frequency (pHF), the ratio of LF to HF (LF/HF) (Preprocessing +FFT), LF, HF, LF/HF (Lomb-Scargle) |
| PPG | 305 | RF 120 EDA-HRV | EDA, PPG | handcrafted | low | no | EDA features: Mean Tonic Component, SD Tonic (Standard deviation of the tonic component), Perc20 (20th percentile of the phasic component), Perc80 Tonic (80th percentile of the tonic component), Quartdev Tonic (Quartile deviation 75 percentile - 25 percentile of the tonic component), Strong Peaks (The number of strong peaks per 100 seconds) and Peaks Phasic (The number of peaks per 100 seconds). HRV features: Time Domain Features: Mean HR, standard deviation of the inter-beat interval (STD RR), mean value of the inter-beat (RR) intervals (Mean RR), root mean square of successive difference of the RR intervals (RMSSD), the percentage of the number of successive RR intervals varying more than 50ms from the previous interval (pNN50), the total number of RR intervals divided by the height of the histogram of all RR intervals measured on a scale with bins of 1/128 s (HRV triangular index), and triangular interpolation of RR interval histogram (TINN). Frequency Domain Features: Low frequency power (LF), high-frequency power (HF), very low frequency power (VLF), prevalent low frequency (pLF), prevalent high frequency (pHF), the ratio of LF to HF (LF/HF) (Preprocessing +FFT), LF, HF, LF/HF (Lomb-Scargle) |
| PPG | 305 | RF 120 HRV | PPG | handcrafted | low | no | Time Domain Features: Mean HR, standard deviation of the inter-beat interval (STD RR), mean value of the inter-beat (RR) intervals (Mean RR), root mean square of successive difference of the RR intervals (RMSSD), the percentage of the number of successive RR intervals varying more than 50ms from the previous interval (pNN50), the total number of RR intervals divided by the height of the histogram of all RR intervals measured on a scale with bins of 1/128 s (HRV triangular index), and triangular interpolation of RR interval histogram (TINN). Frequency Domain Features: Low frequency power (LF), high-frequency power (HF), very low frequency power (VLF), prevalent low frequency (pLF), prevalent high frequency (pHF), the ratio of LF to HF (LF/HF) (Preprocessing +FFT), LF, HF, LF/HF (Lomb-Scargle) |
| PPG | 305 | RF 30 EDA-HRV | EDA, PPG | handcrafted | low | no | EDA features: Mean Tonic Component, SD Tonic (Standard deviation of the tonic component), Perc20 (20th percentile of the phasic component), Perc80 Tonic (80th percentile of the tonic component), Quartdev Tonic (Quartile deviation 75 percentile - 25 percentile of the tonic component), Strong Peaks (The number of strong peaks per 100 seconds) and Peaks Phasic (The number of peaks per 100 seconds). HRV features: Time Domain Features: Mean HR, standard deviation of the inter-beat interval (STD RR), mean value of the inter-beat (RR) intervals (Mean RR), root mean square of successive difference of the RR intervals (RMSSD), the percentage of the number of successive RR intervals varying more than 50ms from the previous interval (pNN50), the total number of RR intervals divided by the height of the histogram of all RR intervals measured on a scale with bins of 1/128 s (HRV triangular index), and triangular interpolation of RR interval histogram (TINN). Frequency Domain Features: Low frequency power (LF), high-frequency power (HF), very low frequency power (VLF), prevalent low frequency (pLF), prevalent high frequency (pHF), the ratio of LF to HF (LF/HF) (Preprocessing +FFT), LF, HF, LF/HF (Lomb-Scargle) |
| PPG | 305 | RF 30 HRV | PPG | handcrafted | low | no | Time Domain Features: Mean HR, standard deviation of the inter-beat interval (STD RR), mean value of the inter-beat (RR) intervals (Mean RR), root mean square of successive difference of the RR intervals (RMSSD), the percentage of the number of successive RR intervals varying more than 50ms from the previous interval (pNN50), the total number of RR intervals divided by the height of the histogram of all RR intervals measured on a scale with bins of 1/128 s (HRV triangular index), and triangular interpolation of RR interval histogram (TINN). Frequency Domain Features: Low frequency power (LF), high-frequency power (HF), very low frequency power (VLF), prevalent low frequency (pLF), prevalent high frequency (pHF), the ratio of LF to HF (LF/HF) (Preprocessing +FFT), LF, HF, LF/HF (Lomb-Scargle) |
| PPG | 305 | RF 60 EDA-HRV | EDA, PPG | handcrafted | low | no | EDA features: Mean Tonic Component, SD Tonic (Standard deviation of the tonic component), Perc20 (20th percentile of the phasic component), Perc80 Tonic (80th percentile of the tonic component), Quartdev Tonic (Quartile deviation 75 percentile - 25 percentile of the tonic component), Strong Peaks (The number of strong peaks per 100 seconds) and Peaks Phasic (The number of peaks per 100 seconds). HRV features: Time Domain Features: Mean HR, standard deviation of the inter-beat interval (STD RR), mean value of the inter-beat (RR) intervals (Mean RR), root mean square of successive difference of the RR intervals (RMSSD), the percentage of the number of successive RR intervals varying more than 50ms from the previous interval (pNN50), the total number of RR intervals divided by the height of the histogram of all RR intervals measured on a scale with bins of 1/128 s (HRV triangular index), and triangular interpolation of RR interval histogram (TINN). Frequency Domain Features: Low frequency power (LF), high-frequency power (HF), very low frequency power (VLF), prevalent low frequency (pLF), prevalent high frequency (pHF), the ratio of LF to HF (LF/HF) (Preprocessing +FFT), LF, HF, LF/HF (Lomb-Scargle) |
| PPG | 305 | RF 60 HRV | PPG | handcrafted | low | no | Time Domain Features: Mean HR, standard deviation of the inter-beat interval (STD RR), mean value of the inter-beat (RR) intervals (Mean RR), root mean square of successive difference of the RR intervals (RMSSD), the percentage of the number of successive RR intervals varying more than 50ms from the previous interval (pNN50), the total number of RR intervals divided by the height of the histogram of all RR intervals measured on a scale with bins of 1/128 s (HRV triangular index), and triangular interpolation of RR interval histogram (TINN). Frequency Domain Features: Low frequency power (LF), high-frequency power (HF), very low frequency power (VLF), prevalent low frequency (pLF), prevalent high frequency (pHF), the ratio of LF to HF (LF/HF) (Preprocessing +FFT), LF, HF, LF/HF (Lomb-Scargle) |
| PPG | 374 | 374.0.1 | PPG | deep | not coded | no | time series data (Automatic feature extraction via DL) |
| PPG | 382 | 382.0.1 HR CNN | PPG | hybrid | medium | no | Statistical Features (mean, standard deviation, variance, coefficient of variance, range, coefficient of range, energy, power, root mean square, first quartile, second quartile or median, third quartile, maximum, skewness, interquartile range, coefficient of interquartile, mean absolute deviation, kurtosis, median absolute deviation, root sum of squares, and signal to noise ratio) |
| PPG | 382 | 382.0.1 HR CRNN | PPG | hybrid | medium | no | Statistical Features (mean, standard deviation, variance, coefficient of variance, range, coefficient of range, energy, power, root mean square, first quartile, second quartile or median, third quartile, maximum, skewness, interquartile range, coefficient of interquartile, mean absolute deviation, kurtosis, median absolute deviation, root sum of squares, and signal to noise ratio) |
| PPG | 382 | 382.0.1 HR RNN | PPG | hybrid | medium | no | Statistical Features (mean, standard deviation, variance, coefficient of variance, range, coefficient of range, energy, power, root mean square, first quartile, second quartile or median, third quartile, maximum, skewness, interquartile range, coefficient of interquartile, mean absolute deviation, kurtosis, median absolute deviation, root sum of squares, and signal to noise ratio) |
| PPG | 382 | 382.0.1 HRB CNN | PPG, microphone | hybrid | medium | no | Statistical Features (mean, standard deviation, variance, coefficient of variance, range, coefficient of range, energy, power, root mean square, first quartile, second quartile or median, third quartile, maximum, skewness, interquartile range, coefficient of interquartile, mean absolute deviation, kurtosis, median absolute deviation, root sum of squares, and signal to noise ratio) + spectral features (Two-dimensional Mel-frequency cepstral coefficient features are transformed into one-dimensional features by condensing along the time axis using the average values) |
| PPG | 382 | 382.0.1 HRB CRNN | PPG, microphone | hybrid | medium | no | Statistical Features (mean, standard deviation, variance, coefficient of variance, range, coefficient of range, energy, power, root mean square, first quartile, second quartile or median, third quartile, maximum, skewness, interquartile range, coefficient of interquartile, mean absolute deviation, kurtosis, median absolute deviation, root sum of squares, and signal to noise ratio) + spectral features (Two-dimensional Mel-frequency cepstral coefficient features are transformed into one-dimensional features by condensing along the time axis using the average values) |
| PPG | 382 | 382.0.1 HRB RNN | PPG, microphone | hybrid | medium | no | Statistical Features (mean, standard deviation, variance, coefficient of variance, range, coefficient of range, energy, power, root mean square, first quartile, second quartile or median, third quartile, maximum, skewness, interquartile range, coefficient of interquartile, mean absolute deviation, kurtosis, median absolute deviation, root sum of squares, and signal to noise ratio) + spectral features (Two-dimensional Mel-frequency cepstral coefficient features are transformed into one-dimensional features by condensing along the time axis using the average values) |
| PPG | 401 | S1 in-sample 1HB | PPG | deep | not coded | no | single wave features for each PPG period by the temporal convolutional network (TCN), temporal relationships with LSTM |
| PPG | 401 | S1 in-sample 30HB s20 | PPG | deep | not coded | no | single wave features for each PPG period by the temporal convolutional network (TCN), temporal relationships with LSTM |
| PPG | 401 | S1 in-sample 30HB s80 | PPG | deep | not coded | no | single wave features for each PPG period by the temporal convolutional network (TCN), temporal relationships with LSTM |
| PPG | 401 | S1 in-sample s80 30HB high-HR | PPG | deep | not coded | no | single wave features for each PPG period by the temporal convolutional network (TCN), temporal relationships with LSTM |
| PPG | 401 | S1 in-sample s80 30HB low-HR | PPG | deep | not coded | no | single wave features for each PPG period by the temporal convolutional network (TCN), temporal relationships with LSTM |
| PPG | 401 | S1 in-sample s80 30HB mid-HR | PPG | deep | not coded | no | single wave features for each PPG period by the temporal convolutional network (TCN), temporal relationships with LSTM |
| PPG | 401 | S1 out-of-sample 15HB | PPG | deep | not coded | no | single wave features for each PPG period by the temporal convolutional network (TCN), temporal relationships with LSTM |
| PPG | 401 | S1 out-of-sample 1HB | PPG | deep | not coded | no | single wave features for each PPG period by the temporal convolutional network (TCN), temporal relationships with LSTM |
| PPG | 401 | S1 out-of-sample 30HB s20 | PPG | deep | not coded | no | single wave features for each PPG period by the temporal convolutional network (TCN), temporal relationships with LSTM |
| PPG | 401 | S1 out-of-sample 30HB s40 | PPG | deep | not coded | no | single wave features for each PPG period by the temporal convolutional network (TCN), temporal relationships with LSTM |
| PPG | 401 | S1 out-of-sample 30HB s60 | PPG | deep | not coded | no | single wave features for each PPG period by the temporal convolutional network (TCN), temporal relationships with LSTM |
| PPG | 401 | S1 out-of-sample 30HB s80 | PPG | deep | not coded | no | single wave features for each PPG period by the temporal convolutional network (TCN), temporal relationships with LSTM |
| PPG | 401 | S1 out-of-sample 5HB | PPG | deep | not coded | no | single wave features for each PPG period by the temporal convolutional network (TCN), temporal relationships with LSTM |
| PPG | 401 | S1 out-of-sample s80 30HB high-HR | PPG | deep | not coded | no | single wave features for each PPG period by the temporal convolutional network (TCN), temporal relationships with LSTM |
| PPG | 401 | S1 out-of-sample s80 30HB low-HR | PPG | deep | not coded | no | single wave features for each PPG period by the temporal convolutional network (TCN), temporal relationships with LSTM |
| PPG | 401 | S1 out-of-sample s80 30HB mid-HR | PPG | deep | not coded | no | single wave features for each PPG period by the temporal convolutional network (TCN), temporal relationships with LSTM |
| PPG | 401 | S2 in-sample s80 30HB high-high | PPG | deep | not coded | no | single wave features for each PPG period by the temporal convolutional network (TCN), temporal relationships with LSTM |
| PPG | 401 | S2 in-sample s80 30HB high-low | PPG | deep | not coded | no | single wave features for each PPG period by the temporal convolutional network (TCN), temporal relationships with LSTM |
| PPG | 401 | S2 in-sample s80 30HB high-mid | PPG | deep | not coded | no | single wave features for each PPG period by the temporal convolutional network (TCN), temporal relationships with LSTM |
| PPG | 401 | S2 in-sample s80 30HB low-high | PPG | deep | not coded | no | single wave features for each PPG period by the temporal convolutional network (TCN), temporal relationships with LSTM |
| PPG | 401 | S2 in-sample s80 30HB low-low | PPG | deep | not coded | no | single wave features for each PPG period by the temporal convolutional network (TCN), temporal relationships with LSTM |
| PPG | 401 | S2 in-sample s80 30HB low-mid | PPG | deep | not coded | no | single wave features for each PPG period by the temporal convolutional network (TCN), temporal relationships with LSTM |
| PPG | 401 | S2 in-sample s80 30HB mid-high | PPG | deep | not coded | no | single wave features for each PPG period by the temporal convolutional network (TCN), temporal relationships with LSTM |
| PPG | 401 | S2 in-sample s80 30HB mid-low | PPG | deep | not coded | no | single wave features for each PPG period by the temporal convolutional network (TCN), temporal relationships with LSTM |
| PPG | 401 | S2 in-sample s80 30HB mid-mid | PPG | deep | not coded | no | single wave features for each PPG period by the temporal convolutional network (TCN), temporal relationships with LSTM |
| PPG | 401 | S2 out-of-sample s80 30HB high-high | PPG | deep | not coded | no | single wave features for each PPG period by the temporal convolutional network (TCN), temporal relationships with LSTM |
| PPG | 401 | S2 out-of-sample s80 30HB high-low | PPG | deep | not coded | no | single wave features for each PPG period by the temporal convolutional network (TCN), temporal relationships with LSTM |
| PPG | 401 | S2 out-of-sample s80 30HB high-mid | PPG | deep | not coded | no | single wave features for each PPG period by the temporal convolutional network (TCN), temporal relationships with LSTM |
| PPG | 401 | S2 out-of-sample s80 30HB low-high | PPG | deep | not coded | no | single wave features for each PPG period by the temporal convolutional network (TCN), temporal relationships with LSTM |
| PPG | 401 | S2 out-of-sample s80 30HB low-low | PPG | deep | not coded | no | single wave features for each PPG period by the temporal convolutional network (TCN), temporal relationships with LSTM |
| PPG | 401 | S2 out-of-sample s80 30HB low-mid | PPG | deep | not coded | no | single wave features for each PPG period by the temporal convolutional network (TCN), temporal relationships with LSTM |
| PPG | 401 | S2 out-of-sample s80 30HB mid-high | PPG | deep | not coded | no | single wave features for each PPG period by the temporal convolutional network (TCN), temporal relationships with LSTM |
| PPG | 401 | S2 out-of-sample s80 30HB mid-low | PPG | deep | not coded | no | single wave features for each PPG period by the temporal convolutional network (TCN), temporal relationships with LSTM |
| PPG | 401 | S2 out-of-sample s80 30HB mid-mid | PPG | deep | not coded | no | single wave features for each PPG period by the temporal convolutional network (TCN), temporal relationships with LSTM |
| PPG | 459 | level 1 | PPG | handcrafted | medium | no | Split into approximation of coefficients and detail coefficients via discrete wavelet decomposition, then split level 1 into high and low frequency signal component. 10 statistical features are extracted from each level of the sub-band levels: Variance, Maximum, Amplitude Minimum Amplitude, Maximum Energy, Minimum Energy, Standard deviation, Peak2peak, Root mean square level(RMS), Mean or Median absolute deviation and Peak magnitude to RMS ratio |
| PPG | 459 | level 2 | PPG | handcrafted | medium | no | Split into approximation of coefficients and detail coefficients via discrete wavelet decomposition, then split level 2 into high and low frequency signal component. 10 statistical features are extracted from each level of the sub-band levels: Variance, Maximum, Amplitude Minimum Amplitude, Maximum Energy, Minimum Energy, Standard deviation, Peak2peak, Root mean square level(RMS), Mean or Median absolute deviation and Peak magnitude to RMS ratio |
| PPG | 459 | level 3 | PPG | handcrafted | medium | no | Split into approximation of coefficients and detail coefficients via discrete wavelet decomposition, then split level 3 into high and low frequency signal component. 10 statistical features are extracted from each level of the sub-band levels: Variance, Maximum, Amplitude Minimum Amplitude, Maximum Energy, Minimum Energy, Standard deviation, Peak2peak, Root mean square level(RMS), Mean or Median absolute deviation and Peak magnitude to RMS ratio |
| PPG | 459 | level 4 | PPG | handcrafted | medium | no | Split into approximation of coefficients and detail coefficients via discrete wavelet decomposition, then split level 4 into high and low frequency signal component. 10 statistical features are extracted from each level of the sub-band levels: Variance, Maximum, Amplitude Minimum Amplitude, Maximum Energy, Minimum Energy, Standard deviation, Peak2peak, Root mean square level(RMS), Mean or Median absolute deviation and Peak magnitude to RMS ratio |
| PPG | 459 | level fusion | PPG | handcrafted | medium | no | Split into approximation of coefficients and detail coefficients via discrete wavelet decomposition, then each n level into high and low frequency signal component. 10 statistical features are extracted from each level of the sub-band levels: Variance, Maximum, Amplitude Minimum Amplitude, Maximum Energy, Minimum Energy, Standard deviation, Peak2peak, Root mean square level(RMS), Mean or Median absolute deviation and Peak magnitude to RMS ratio |
| PPG | 486 | 486.0.1 / Exp 2 Bior1.1 A1 | PPG | handcrafted | low | no | time-domain statistical features from wavelet-decomposed signal into 3 sub-bands (Variance, Mean energy, Minimum energy, Maximum energy, Mean, Minimum amplitude, Standard deviation (STD), Maximum amplitude, Range, Peak-to-peak, Root‑mean‑square (RMS), Peak magnitude to RMS ratio) from each detail (D1–D3) and approximation (A1–A3) |
| PPG | 486 | 486.0.1 / Exp 2 Bior1.1 A2 | PPG | handcrafted | low | no | time-domain statistical features from wavelet-decomposed signal into 3 sub-bands (Variance, Mean energy, Minimum energy, Maximum energy, Mean, Minimum amplitude, Standard deviation (STD), Maximum amplitude, Range, Peak-to-peak, Root‑mean‑square (RMS), Peak magnitude to RMS ratio) from each detail (D1–D3) and approximation (A1–A3) |
| PPG | 486 | 486.0.1 / Exp 2 Bior1.1 A3 | PPG | handcrafted | low | no | time-domain statistical features from wavelet-decomposed signal into 3 sub-bands (Variance, Mean energy, Minimum energy, Maximum energy, Mean, Minimum amplitude, Standard deviation (STD), Maximum amplitude, Range, Peak-to-peak, Root‑mean‑square (RMS), Peak magnitude to RMS ratio) from each detail (D1–D3) and approximation (A1–A3) |
| PPG | 486 | 486.0.1 / Exp 2 Bior1.1 D1 | PPG | handcrafted | low | no | time-domain statistical features from wavelet-decomposed signal into 3 sub-bands (Variance, Mean energy, Minimum energy, Maximum energy, Mean, Minimum amplitude, Standard deviation (STD), Maximum amplitude, Range, Peak-to-peak, Root‑mean‑square (RMS), Peak magnitude to RMS ratio) from each detail (D1–D3) and approximation (A1–A3) |
| PPG | 486 | 486.0.1 / Exp 2 Bior1.1 D2 | PPG | handcrafted | low | no | time-domain statistical features from wavelet-decomposed signal into 3 sub-bands (Variance, Mean energy, Minimum energy, Maximum energy, Mean, Minimum amplitude, Standard deviation (STD), Maximum amplitude, Range, Peak-to-peak, Root‑mean‑square (RMS), Peak magnitude to RMS ratio) from each detail (D1–D3) and approximation (A1–A3) |
| PPG | 486 | 486.0.1 / Exp 2 Bior1.1 D3 | PPG | handcrafted | low | no | time-domain statistical features from wavelet-decomposed signal into 3 sub-bands (Variance, Mean energy, Minimum energy, Maximum energy, Mean, Minimum amplitude, Standard deviation (STD), Maximum amplitude, Range, Peak-to-peak, Root‑mean‑square (RMS), Peak magnitude to RMS ratio) from each detail (D1–D3) and approximation (A1–A3) |
| PPG | 486 | 486.0.1 / Exp 2 Bior1.3 A1 | PPG | handcrafted | low | no | time-domain statistical features from wavelet-decomposed signal into 3 sub-bands (Variance, Mean energy, Minimum energy, Maximum energy, Mean, Minimum amplitude, Standard deviation (STD), Maximum amplitude, Range, Peak-to-peak, Root‑mean‑square (RMS), Peak magnitude to RMS ratio) from each detail (D1–D3) and approximation (A1–A3) |
| PPG | 486 | 486.0.1 / Exp 2 Bior1.3 A2 | PPG | handcrafted | low | no | time-domain statistical features from wavelet-decomposed signal into 3 sub-bands (Variance, Mean energy, Minimum energy, Maximum energy, Mean, Minimum amplitude, Standard deviation (STD), Maximum amplitude, Range, Peak-to-peak, Root‑mean‑square (RMS), Peak magnitude to RMS ratio) from each detail (D1–D3) and approximation (A1–A3) |
| PPG | 486 | 486.0.1 / Exp 2 Bior1.3 A3 | PPG | handcrafted | low | no | time-domain statistical features from wavelet-decomposed signal into 3 sub-bands (Variance, Mean energy, Minimum energy, Maximum energy, Mean, Minimum amplitude, Standard deviation (STD), Maximum amplitude, Range, Peak-to-peak, Root‑mean‑square (RMS), Peak magnitude to RMS ratio) from each detail (D1–D3) and approximation (A1–A3) |
| PPG | 486 | 486.0.1 / Exp 2 Bior1.3 D1 | PPG | handcrafted | low | no | time-domain statistical features from wavelet-decomposed signal into 3 sub-bands (Variance, Mean energy, Minimum energy, Maximum energy, Mean, Minimum amplitude, Standard deviation (STD), Maximum amplitude, Range, Peak-to-peak, Root‑mean‑square (RMS), Peak magnitude to RMS ratio) from each detail (D1–D3) and approximation (A1–A3) |
| PPG | 486 | 486.0.1 / Exp 2 Bior1.3 D2 | PPG | handcrafted | low | no | time-domain statistical features from wavelet-decomposed signal into 3 sub-bands (Variance, Mean energy, Minimum energy, Maximum energy, Mean, Minimum amplitude, Standard deviation (STD), Maximum amplitude, Range, Peak-to-peak, Root‑mean‑square (RMS), Peak magnitude to RMS ratio) from each detail (D1–D3) and approximation (A1–A3) |
| PPG | 486 | 486.0.1 / Exp 2 Bior1.3 D3 | PPG | handcrafted | low | no | time-domain statistical features from wavelet-decomposed signal into 3 sub-bands (Variance, Mean energy, Minimum energy, Maximum energy, Mean, Minimum amplitude, Standard deviation (STD), Maximum amplitude, Range, Peak-to-peak, Root‑mean‑square (RMS), Peak magnitude to RMS ratio) from each detail (D1–D3) and approximation (A1–A3) |
| PPG | 486 | 486.0.1 / Exp 2 Bior1.5 A1 | PPG | handcrafted | low | no | time-domain statistical features from wavelet-decomposed signal into 3 sub-bands (Variance, Mean energy, Minimum energy, Maximum energy, Mean, Minimum amplitude, Standard deviation (STD), Maximum amplitude, Range, Peak-to-peak, Root‑mean‑square (RMS), Peak magnitude to RMS ratio) from each detail (D1–D3) and approximation (A1–A3) |
| PPG | 486 | 486.0.1 / Exp 2 Bior1.5 A2 | PPG | handcrafted | low | no | time-domain statistical features from wavelet-decomposed signal into 3 sub-bands (Variance, Mean energy, Minimum energy, Maximum energy, Mean, Minimum amplitude, Standard deviation (STD), Maximum amplitude, Range, Peak-to-peak, Root‑mean‑square (RMS), Peak magnitude to RMS ratio) from each detail (D1–D3) and approximation (A1–A3) |
| PPG | 486 | 486.0.1 / Exp 2 Bior1.5 A3 | PPG | handcrafted | low | no | time-domain statistical features from wavelet-decomposed signal into 3 sub-bands (Variance, Mean energy, Minimum energy, Maximum energy, Mean, Minimum amplitude, Standard deviation (STD), Maximum amplitude, Range, Peak-to-peak, Root‑mean‑square (RMS), Peak magnitude to RMS ratio) from each detail (D1–D3) and approximation (A1–A3) |
| PPG | 486 | 486.0.1 / Exp 2 Bior1.5 D1 | PPG | handcrafted | low | no | time-domain statistical features from wavelet-decomposed signal into 3 sub-bands (Variance, Mean energy, Minimum energy, Maximum energy, Mean, Minimum amplitude, Standard deviation (STD), Maximum amplitude, Range, Peak-to-peak, Root‑mean‑square (RMS), Peak magnitude to RMS ratio) from each detail (D1–D3) and approximation (A1–A3) |
| PPG | 486 | 486.0.1 / Exp 2 Bior1.5 D2 | PPG | handcrafted | low | no | time-domain statistical features from wavelet-decomposed signal into 3 sub-bands (Variance, Mean energy, Minimum energy, Maximum energy, Mean, Minimum amplitude, Standard deviation (STD), Maximum amplitude, Range, Peak-to-peak, Root‑mean‑square (RMS), Peak magnitude to RMS ratio) from each detail (D1–D3) and approximation (A1–A3) |
| PPG | 486 | 486.0.1 / Exp 2 Bior1.5 D3 | PPG | handcrafted | low | no | time-domain statistical features from wavelet-decomposed signal into 3 sub-bands (Variance, Mean energy, Minimum energy, Maximum energy, Mean, Minimum amplitude, Standard deviation (STD), Maximum amplitude, Range, Peak-to-peak, Root‑mean‑square (RMS), Peak magnitude to RMS ratio) from each detail (D1–D3) and approximation (A1–A3) |
| PPG | 486 | 486.0.1 / Exp 2 Bior2.2 D1 | PPG | handcrafted | low | no | time-domain statistical features from wavelet-decomposed signal into 3 sub-bands (Variance, Mean energy, Minimum energy, Maximum energy, Mean, Minimum amplitude, Standard deviation (STD), Maximum amplitude, Range, Peak-to-peak, Root‑mean‑square (RMS), Peak magnitude to RMS ratio) from each detail (D1–D3) and approximation (A1–A3) |
| PPG | 486 | 486.0.1 / Exp 2 Bior2.2 A1 | PPG | handcrafted | low | no | time-domain statistical features from wavelet-decomposed signal into 3 sub-bands (Variance, Mean energy, Minimum energy, Maximum energy, Mean, Minimum amplitude, Standard deviation (STD), Maximum amplitude, Range, Peak-to-peak, Root‑mean‑square (RMS), Peak magnitude to RMS ratio) from each detail (D1–D3) and approximation (A1–A3) |
| PPG | 486 | 486.0.1 / Exp 2 Bior2.2 A2 | PPG | handcrafted | low | no | time-domain statistical features from wavelet-decomposed signal into 3 sub-bands (Variance, Mean energy, Minimum energy, Maximum energy, Mean, Minimum amplitude, Standard deviation (STD), Maximum amplitude, Range, Peak-to-peak, Root‑mean‑square (RMS), Peak magnitude to RMS ratio) from each detail (D1–D3) and approximation (A1–A3) |
| PPG | 486 | 486.0.1 / Exp 2 Bior2.2 A3 | PPG | handcrafted | low | no | time-domain statistical features from wavelet-decomposed signal into 3 sub-bands (Variance, Mean energy, Minimum energy, Maximum energy, Mean, Minimum amplitude, Standard deviation (STD), Maximum amplitude, Range, Peak-to-peak, Root‑mean‑square (RMS), Peak magnitude to RMS ratio) from each detail (D1–D3) and approximation (A1–A3) |
| PPG | 486 | 486.0.1 / Exp 2 Bior2.2 D2 | PPG | handcrafted | low | no | time-domain statistical features from wavelet-decomposed signal into 3 sub-bands (Variance, Mean energy, Minimum energy, Maximum energy, Mean, Minimum amplitude, Standard deviation (STD), Maximum amplitude, Range, Peak-to-peak, Root‑mean‑square (RMS), Peak magnitude to RMS ratio) from each detail (D1–D3) and approximation (A1–A3) |
| PPG | 486 | 486.0.1 / Exp 2 Bior2.2 D3 | PPG | handcrafted | low | no | time-domain statistical features from wavelet-decomposed signal into 3 sub-bands (Variance, Mean energy, Minimum energy, Maximum energy, Mean, Minimum amplitude, Standard deviation (STD), Maximum amplitude, Range, Peak-to-peak, Root‑mean‑square (RMS), Peak magnitude to RMS ratio) from each detail (D1–D3) and approximation (A1–A3) |
| PPG | 486 | 486.0.1 / Exp 2 Bior2.4 D1 | PPG | handcrafted | low | no | time-domain statistical features from wavelet-decomposed signal into 3 sub-bands (Variance, Mean energy, Minimum energy, Maximum energy, Mean, Minimum amplitude, Standard deviation (STD), Maximum amplitude, Range, Peak-to-peak, Root‑mean‑square (RMS), Peak magnitude to RMS ratio) from each detail (D1–D3) and approximation (A1–A3) |
| PPG | 486 | 486.0.1 / Exp 2 Bior2.4 A1 | PPG | handcrafted | low | no | time-domain statistical features from wavelet-decomposed signal into 3 sub-bands (Variance, Mean energy, Minimum energy, Maximum energy, Mean, Minimum amplitude, Standard deviation (STD), Maximum amplitude, Range, Peak-to-peak, Root‑mean‑square (RMS), Peak magnitude to RMS ratio) from each detail (D1–D3) and approximation (A1–A3) |
| PPG | 486 | 486.0.1 / Exp 2 Bior2.4 A2 | PPG | handcrafted | low | no | time-domain statistical features from wavelet-decomposed signal into 3 sub-bands (Variance, Mean energy, Minimum energy, Maximum energy, Mean, Minimum amplitude, Standard deviation (STD), Maximum amplitude, Range, Peak-to-peak, Root‑mean‑square (RMS), Peak magnitude to RMS ratio) from each detail (D1–D3) and approximation (A1–A3) |
| PPG | 486 | 486.0.1 / Exp 2 Bior2.4 A3 | PPG | handcrafted | low | no | time-domain statistical features from wavelet-decomposed signal into 3 sub-bands (Variance, Mean energy, Minimum energy, Maximum energy, Mean, Minimum amplitude, Standard deviation (STD), Maximum amplitude, Range, Peak-to-peak, Root‑mean‑square (RMS), Peak magnitude to RMS ratio) from each detail (D1–D3) and approximation (A1–A3) |
| PPG | 486 | 486.0.1 / Exp 2 Bior2.4 D2 | PPG | handcrafted | low | no | time-domain statistical features from wavelet-decomposed signal into 3 sub-bands (Variance, Mean energy, Minimum energy, Maximum energy, Mean, Minimum amplitude, Standard deviation (STD), Maximum amplitude, Range, Peak-to-peak, Root‑mean‑square (RMS), Peak magnitude to RMS ratio) from each detail (D1–D3) and approximation (A1–A3) |
| PPG | 486 | 486.0.1 / Exp 2 Bior2.4 D3 | PPG | handcrafted | low | no | time-domain statistical features from wavelet-decomposed signal into 3 sub-bands (Variance, Mean energy, Minimum energy, Maximum energy, Mean, Minimum amplitude, Standard deviation (STD), Maximum amplitude, Range, Peak-to-peak, Root‑mean‑square (RMS), Peak magnitude to RMS ratio) from each detail (D1–D3) and approximation (A1–A3) |
| PPG | 486 | 486.0.1 / Exp 2 Bior2.6 D1 | PPG | handcrafted | low | no | time-domain statistical features from wavelet-decomposed signal into 3 sub-bands (Variance, Mean energy, Minimum energy, Maximum energy, Mean, Minimum amplitude, Standard deviation (STD), Maximum amplitude, Range, Peak-to-peak, Root‑mean‑square (RMS), Peak magnitude to RMS ratio) from each detail (D1–D3) and approximation (A1–A3) |
| PPG | 486 | 486.0.1 / Exp 2 Bior2.6 A1 | PPG | handcrafted | low | no | time-domain statistical features from wavelet-decomposed signal into 3 sub-bands (Variance, Mean energy, Minimum energy, Maximum energy, Mean, Minimum amplitude, Standard deviation (STD), Maximum amplitude, Range, Peak-to-peak, Root‑mean‑square (RMS), Peak magnitude to RMS ratio) from each detail (D1–D3) and approximation (A1–A3) |
| PPG | 486 | 486.0.1 / Exp 2 Bior2.6 A2 | PPG | handcrafted | low | no | time-domain statistical features from wavelet-decomposed signal into 3 sub-bands (Variance, Mean energy, Minimum energy, Maximum energy, Mean, Minimum amplitude, Standard deviation (STD), Maximum amplitude, Range, Peak-to-peak, Root‑mean‑square (RMS), Peak magnitude to RMS ratio) from each detail (D1–D3) and approximation (A1–A3) |
| PPG | 486 | 486.0.1 / Exp 2 Bior2.6 A3 | PPG | handcrafted | low | no | time-domain statistical features from wavelet-decomposed signal into 3 sub-bands (Variance, Mean energy, Minimum energy, Maximum energy, Mean, Minimum amplitude, Standard deviation (STD), Maximum amplitude, Range, Peak-to-peak, Root‑mean‑square (RMS), Peak magnitude to RMS ratio) from each detail (D1–D3) and approximation (A1–A3) |
| PPG | 486 | 486.0.1 / Exp 2 Bior2.6 D2 | PPG | handcrafted | low | no | time-domain statistical features from wavelet-decomposed signal into 3 sub-bands (Variance, Mean energy, Minimum energy, Maximum energy, Mean, Minimum amplitude, Standard deviation (STD), Maximum amplitude, Range, Peak-to-peak, Root‑mean‑square (RMS), Peak magnitude to RMS ratio) from each detail (D1–D3) and approximation (A1–A3) |
| PPG | 486 | 486.0.1 / Exp 2 Bior2.6 D3 | PPG | handcrafted | low | no | time-domain statistical features from wavelet-decomposed signal into 3 sub-bands (Variance, Mean energy, Minimum energy, Maximum energy, Mean, Minimum amplitude, Standard deviation (STD), Maximum amplitude, Range, Peak-to-peak, Root‑mean‑square (RMS), Peak magnitude to RMS ratio) from each detail (D1–D3) and approximation (A1–A3) |
| PPG | 486 | 486.0.1 / Exp 2 Bior2.8 D1 | PPG | handcrafted | low | no | time-domain statistical features from wavelet-decomposed signal into 3 sub-bands (Variance, Mean energy, Minimum energy, Maximum energy, Mean, Minimum amplitude, Standard deviation (STD), Maximum amplitude, Range, Peak-to-peak, Root‑mean‑square (RMS), Peak magnitude to RMS ratio) from each detail (D1–D3) and approximation (A1–A3) |
| PPG | 486 | 486.0.1 / Exp 2 Bior2.8 A1 | PPG | handcrafted | low | no | time-domain statistical features from wavelet-decomposed signal into 3 sub-bands (Variance, Mean energy, Minimum energy, Maximum energy, Mean, Minimum amplitude, Standard deviation (STD), Maximum amplitude, Range, Peak-to-peak, Root‑mean‑square (RMS), Peak magnitude to RMS ratio) from each detail (D1–D3) and approximation (A1–A3) |
| PPG | 486 | 486.0.1 / Exp 2 Bior2.8 A2 | PPG | handcrafted | low | no | time-domain statistical features from wavelet-decomposed signal into 3 sub-bands (Variance, Mean energy, Minimum energy, Maximum energy, Mean, Minimum amplitude, Standard deviation (STD), Maximum amplitude, Range, Peak-to-peak, Root‑mean‑square (RMS), Peak magnitude to RMS ratio) from each detail (D1–D3) and approximation (A1–A3) |
| PPG | 486 | 486.0.1 / Exp 2 Bior2.8 A3 | PPG | handcrafted | low | no | time-domain statistical features from wavelet-decomposed signal into 3 sub-bands (Variance, Mean energy, Minimum energy, Maximum energy, Mean, Minimum amplitude, Standard deviation (STD), Maximum amplitude, Range, Peak-to-peak, Root‑mean‑square (RMS), Peak magnitude to RMS ratio) from each detail (D1–D3) and approximation (A1–A3) |
| PPG | 486 | 486.0.1 / Exp 2 Bior2.8 D2 | PPG | handcrafted | low | no | time-domain statistical features from wavelet-decomposed signal into 3 sub-bands (Variance, Mean energy, Minimum energy, Maximum energy, Mean, Minimum amplitude, Standard deviation (STD), Maximum amplitude, Range, Peak-to-peak, Root‑mean‑square (RMS), Peak magnitude to RMS ratio) from each detail (D1–D3) and approximation (A1–A3) |
| PPG | 486 | 486.0.1 / Exp 2 Bior2.8 D3 | PPG | handcrafted | low | no | time-domain statistical features from wavelet-decomposed signal into 3 sub-bands (Variance, Mean energy, Minimum energy, Maximum energy, Mean, Minimum amplitude, Standard deviation (STD), Maximum amplitude, Range, Peak-to-peak, Root‑mean‑square (RMS), Peak magnitude to RMS ratio) from each detail (D1–D3) and approximation (A1–A3) |
| PPG | 486 | 486.0.1 / Exp 2 Bior3.1 A1 | PPG | handcrafted | low | no | time-domain statistical features from wavelet-decomposed signal into 3 sub-bands (Variance, Mean energy, Minimum energy, Maximum energy, Mean, Minimum amplitude, Standard deviation (STD), Maximum amplitude, Range, Peak-to-peak, Root‑mean‑square (RMS), Peak magnitude to RMS ratio) from each detail (D1–D3) and approximation (A1–A3) |
| PPG | 486 | 486.0.1 / Exp 2 Bior3.1 A2 | PPG | handcrafted | low | no | time-domain statistical features from wavelet-decomposed signal into 3 sub-bands (Variance, Mean energy, Minimum energy, Maximum energy, Mean, Minimum amplitude, Standard deviation (STD), Maximum amplitude, Range, Peak-to-peak, Root‑mean‑square (RMS), Peak magnitude to RMS ratio) from each detail (D1–D3) and approximation (A1–A3) |
| PPG | 486 | 486.0.1 / Exp 2 Bior3.1 A3 | PPG | handcrafted | low | no | time-domain statistical features from wavelet-decomposed signal into 3 sub-bands (Variance, Mean energy, Minimum energy, Maximum energy, Mean, Minimum amplitude, Standard deviation (STD), Maximum amplitude, Range, Peak-to-peak, Root‑mean‑square (RMS), Peak magnitude to RMS ratio) from each detail (D1–D3) and approximation (A1–A3) |
| PPG | 486 | 486.0.1 / Exp 2 Bior3.1 D1 | PPG | handcrafted | low | no | time-domain statistical features from wavelet-decomposed signal into 3 sub-bands (Variance, Mean energy, Minimum energy, Maximum energy, Mean, Minimum amplitude, Standard deviation (STD), Maximum amplitude, Range, Peak-to-peak, Root‑mean‑square (RMS), Peak magnitude to RMS ratio) from each detail (D1–D3) and approximation (A1–A3) |
| PPG | 486 | 486.0.1 / Exp 2 Bior3.1 D2 | PPG | handcrafted | low | no | time-domain statistical features from wavelet-decomposed signal into 3 sub-bands (Variance, Mean energy, Minimum energy, Maximum energy, Mean, Minimum amplitude, Standard deviation (STD), Maximum amplitude, Range, Peak-to-peak, Root‑mean‑square (RMS), Peak magnitude to RMS ratio) from each detail (D1–D3) and approximation (A1–A3) |
| PPG | 486 | 486.0.1 / Exp 2 Bior3.1 D3 | PPG | handcrafted | low | no | time-domain statistical features from wavelet-decomposed signal into 3 sub-bands (Variance, Mean energy, Minimum energy, Maximum energy, Mean, Minimum amplitude, Standard deviation (STD), Maximum amplitude, Range, Peak-to-peak, Root‑mean‑square (RMS), Peak magnitude to RMS ratio) from each detail (D1–D3) and approximation (A1–A3) |
| PPG | 486 | 486.0.1 / Exp 2 Bior3.3 A1 | PPG | handcrafted | low | no | time-domain statistical features from wavelet-decomposed signal into 3 sub-bands (Variance, Mean energy, Minimum energy, Maximum energy, Mean, Minimum amplitude, Standard deviation (STD), Maximum amplitude, Range, Peak-to-peak, Root‑mean‑square (RMS), Peak magnitude to RMS ratio) from each detail (D1–D3) and approximation (A1–A3) |
| PPG | 486 | 486.0.1 / Exp 2 Bior3.3 A2 | PPG | handcrafted | low | no | time-domain statistical features from wavelet-decomposed signal into 3 sub-bands (Variance, Mean energy, Minimum energy, Maximum energy, Mean, Minimum amplitude, Standard deviation (STD), Maximum amplitude, Range, Peak-to-peak, Root‑mean‑square (RMS), Peak magnitude to RMS ratio) from each detail (D1–D3) and approximation (A1–A3) |
| PPG | 486 | 486.0.1 / Exp 2 Bior3.3 A3 | PPG | handcrafted | low | no | time-domain statistical features from wavelet-decomposed signal into 3 sub-bands (Variance, Mean energy, Minimum energy, Maximum energy, Mean, Minimum amplitude, Standard deviation (STD), Maximum amplitude, Range, Peak-to-peak, Root‑mean‑square (RMS), Peak magnitude to RMS ratio) from each detail (D1–D3) and approximation (A1–A3) |
| PPG | 486 | 486.0.1 / Exp 2 Bior3.3 D1 | PPG | handcrafted | low | no | time-domain statistical features from wavelet-decomposed signal into 3 sub-bands (Variance, Mean energy, Minimum energy, Maximum energy, Mean, Minimum amplitude, Standard deviation (STD), Maximum amplitude, Range, Peak-to-peak, Root‑mean‑square (RMS), Peak magnitude to RMS ratio) from each detail (D1–D3) and approximation (A1–A3) |
| PPG | 486 | 486.0.1 / Exp 2 Bior3.3 D2 | PPG | handcrafted | low | no | time-domain statistical features from wavelet-decomposed signal into 3 sub-bands (Variance, Mean energy, Minimum energy, Maximum energy, Mean, Minimum amplitude, Standard deviation (STD), Maximum amplitude, Range, Peak-to-peak, Root‑mean‑square (RMS), Peak magnitude to RMS ratio) from each detail (D1–D3) and approximation (A1–A3) |
| PPG | 486 | 486.0.1 / Exp 2 Bior3.3 D3 | PPG | handcrafted | low | no | time-domain statistical features from wavelet-decomposed signal into 3 sub-bands (Variance, Mean energy, Minimum energy, Maximum energy, Mean, Minimum amplitude, Standard deviation (STD), Maximum amplitude, Range, Peak-to-peak, Root‑mean‑square (RMS), Peak magnitude to RMS ratio) from each detail (D1–D3) and approximation (A1–A3) |
| PPG | 486 | 486.0.1 / Exp 2 Bior3.5 A1 | PPG | handcrafted | low | no | time-domain statistical features from wavelet-decomposed signal into 3 sub-bands (Variance, Mean energy, Minimum energy, Maximum energy, Mean, Minimum amplitude, Standard deviation (STD), Maximum amplitude, Range, Peak-to-peak, Root‑mean‑square (RMS), Peak magnitude to RMS ratio) from each detail (D1–D3) and approximation (A1–A3) |
| PPG | 486 | 486.0.1 / Exp 2 Bior3.5 A2 | PPG | handcrafted | low | no | time-domain statistical features from wavelet-decomposed signal into 3 sub-bands (Variance, Mean energy, Minimum energy, Maximum energy, Mean, Minimum amplitude, Standard deviation (STD), Maximum amplitude, Range, Peak-to-peak, Root‑mean‑square (RMS), Peak magnitude to RMS ratio) from each detail (D1–D3) and approximation (A1–A3) |
| PPG | 486 | 486.0.1 / Exp 2 Bior3.5 A3 | PPG | handcrafted | low | no | time-domain statistical features from wavelet-decomposed signal into 3 sub-bands (Variance, Mean energy, Minimum energy, Maximum energy, Mean, Minimum amplitude, Standard deviation (STD), Maximum amplitude, Range, Peak-to-peak, Root‑mean‑square (RMS), Peak magnitude to RMS ratio) from each detail (D1–D3) and approximation (A1–A3) |
| PPG | 486 | 486.0.1 / Exp 2 Bior3.5 D1 | PPG | handcrafted | low | no | time-domain statistical features from wavelet-decomposed signal into 3 sub-bands (Variance, Mean energy, Minimum energy, Maximum energy, Mean, Minimum amplitude, Standard deviation (STD), Maximum amplitude, Range, Peak-to-peak, Root‑mean‑square (RMS), Peak magnitude to RMS ratio) from each detail (D1–D3) and approximation (A1–A3) |
| PPG | 486 | 486.0.1 / Exp 2 Bior3.5 D2 | PPG | handcrafted | low | no | time-domain statistical features from wavelet-decomposed signal into 3 sub-bands (Variance, Mean energy, Minimum energy, Maximum energy, Mean, Minimum amplitude, Standard deviation (STD), Maximum amplitude, Range, Peak-to-peak, Root‑mean‑square (RMS), Peak magnitude to RMS ratio) from each detail (D1–D3) and approximation (A1–A3) |
| PPG | 486 | 486.0.1 / Exp 2 Bior3.5 D3 | PPG | handcrafted | low | no | time-domain statistical features from wavelet-decomposed signal into 3 sub-bands (Variance, Mean energy, Minimum energy, Maximum energy, Mean, Minimum amplitude, Standard deviation (STD), Maximum amplitude, Range, Peak-to-peak, Root‑mean‑square (RMS), Peak magnitude to RMS ratio) from each detail (D1–D3) and approximation (A1–A3) |
| PPG | 486 | 486.0.1 / Exp 2 Bior3.7 A1 | PPG | handcrafted | low | no | time-domain statistical features from wavelet-decomposed signal into 3 sub-bands (Variance, Mean energy, Minimum energy, Maximum energy, Mean, Minimum amplitude, Standard deviation (STD), Maximum amplitude, Range, Peak-to-peak, Root‑mean‑square (RMS), Peak magnitude to RMS ratio) from each detail (D1–D3) and approximation (A1–A3) |
| PPG | 486 | 486.0.1 / Exp 2 Bior3.7 A2 | PPG | handcrafted | low | no | time-domain statistical features from wavelet-decomposed signal into 3 sub-bands (Variance, Mean energy, Minimum energy, Maximum energy, Mean, Minimum amplitude, Standard deviation (STD), Maximum amplitude, Range, Peak-to-peak, Root‑mean‑square (RMS), Peak magnitude to RMS ratio) from each detail (D1–D3) and approximation (A1–A3) |
| PPG | 486 | 486.0.1 / Exp 2 Bior3.7 A3 | PPG | handcrafted | low | no | time-domain statistical features from wavelet-decomposed signal into 3 sub-bands (Variance, Mean energy, Minimum energy, Maximum energy, Mean, Minimum amplitude, Standard deviation (STD), Maximum amplitude, Range, Peak-to-peak, Root‑mean‑square (RMS), Peak magnitude to RMS ratio) from each detail (D1–D3) and approximation (A1–A3) |
| PPG | 486 | 486.0.1 / Exp 2 Bior3.7 D1 | PPG | handcrafted | low | no | time-domain statistical features from wavelet-decomposed signal into 3 sub-bands (Variance, Mean energy, Minimum energy, Maximum energy, Mean, Minimum amplitude, Standard deviation (STD), Maximum amplitude, Range, Peak-to-peak, Root‑mean‑square (RMS), Peak magnitude to RMS ratio) from each detail (D1–D3) and approximation (A1–A3) |
| PPG | 486 | 486.0.1 / Exp 2 Bior3.7 D2 | PPG | handcrafted | low | no | time-domain statistical features from wavelet-decomposed signal into 3 sub-bands (Variance, Mean energy, Minimum energy, Maximum energy, Mean, Minimum amplitude, Standard deviation (STD), Maximum amplitude, Range, Peak-to-peak, Root‑mean‑square (RMS), Peak magnitude to RMS ratio) from each detail (D1–D3) and approximation (A1–A3) |
| PPG | 486 | 486.0.1 / Exp 2 Bior3.7 D3 | PPG | handcrafted | low | no | time-domain statistical features from wavelet-decomposed signal into 3 sub-bands (Variance, Mean energy, Minimum energy, Maximum energy, Mean, Minimum amplitude, Standard deviation (STD), Maximum amplitude, Range, Peak-to-peak, Root‑mean‑square (RMS), Peak magnitude to RMS ratio) from each detail (D1–D3) and approximation (A1–A3) |
| PPG | 486 | 486.0.1 / Exp 2 Bior3.9 A1 | PPG | handcrafted | low | no | time-domain statistical features from wavelet-decomposed signal into 3 sub-bands (Variance, Mean energy, Minimum energy, Maximum energy, Mean, Minimum amplitude, Standard deviation (STD), Maximum amplitude, Range, Peak-to-peak, Root‑mean‑square (RMS), Peak magnitude to RMS ratio) from each detail (D1–D3) and approximation (A1–A3) |
| PPG | 486 | 486.0.1 / Exp 2 Bior3.9 A2 | PPG | handcrafted | low | no | time-domain statistical features from wavelet-decomposed signal into 3 sub-bands (Variance, Mean energy, Minimum energy, Maximum energy, Mean, Minimum amplitude, Standard deviation (STD), Maximum amplitude, Range, Peak-to-peak, Root‑mean‑square (RMS), Peak magnitude to RMS ratio) from each detail (D1–D3) and approximation (A1–A3) |
| PPG | 486 | 486.0.1 / Exp 2 Bior3.9 A3 | PPG | handcrafted | low | no | time-domain statistical features from wavelet-decomposed signal into 3 sub-bands (Variance, Mean energy, Minimum energy, Maximum energy, Mean, Minimum amplitude, Standard deviation (STD), Maximum amplitude, Range, Peak-to-peak, Root‑mean‑square (RMS), Peak magnitude to RMS ratio) from each detail (D1–D3) and approximation (A1–A3) |
| PPG | 486 | 486.0.1 / Exp 2 Bior3.9 D1 | PPG | handcrafted | low | no | time-domain statistical features from wavelet-decomposed signal into 3 sub-bands (Variance, Mean energy, Minimum energy, Maximum energy, Mean, Minimum amplitude, Standard deviation (STD), Maximum amplitude, Range, Peak-to-peak, Root‑mean‑square (RMS), Peak magnitude to RMS ratio) from each detail (D1–D3) and approximation (A1–A3) |
| PPG | 486 | 486.0.1 / Exp 2 Bior3.9 D2 | PPG | handcrafted | low | no | time-domain statistical features from wavelet-decomposed signal into 3 sub-bands (Variance, Mean energy, Minimum energy, Maximum energy, Mean, Minimum amplitude, Standard deviation (STD), Maximum amplitude, Range, Peak-to-peak, Root‑mean‑square (RMS), Peak magnitude to RMS ratio) from each detail (D1–D3) and approximation (A1–A3) |
| PPG | 486 | 486.0.1 / Exp 2 Bior3.9 D3 | PPG | handcrafted | low | no | time-domain statistical features from wavelet-decomposed signal into 3 sub-bands (Variance, Mean energy, Minimum energy, Maximum energy, Mean, Minimum amplitude, Standard deviation (STD), Maximum amplitude, Range, Peak-to-peak, Root‑mean‑square (RMS), Peak magnitude to RMS ratio) from each detail (D1–D3) and approximation (A1–A3) |
| PPG | 486 | 486.0.1 / Exp 2 Bior4.4 A1 | PPG | handcrafted | low | no | time-domain statistical features from wavelet-decomposed signal into 3 sub-bands (Variance, Mean energy, Minimum energy, Maximum energy, Mean, Minimum amplitude, Standard deviation (STD), Maximum amplitude, Range, Peak-to-peak, Root‑mean‑square (RMS), Peak magnitude to RMS ratio) from each detail (D1–D3) and approximation (A1–A3) |
| PPG | 486 | 486.0.1 / Exp 2 Bior4.4 A2 | PPG | handcrafted | low | no | time-domain statistical features from wavelet-decomposed signal into 3 sub-bands (Variance, Mean energy, Minimum energy, Maximum energy, Mean, Minimum amplitude, Standard deviation (STD), Maximum amplitude, Range, Peak-to-peak, Root‑mean‑square (RMS), Peak magnitude to RMS ratio) from each detail (D1–D3) and approximation (A1–A3) |
| PPG | 486 | 486.0.1 / Exp 2 Bior4.4 A3 | PPG | handcrafted | low | no | time-domain statistical features from wavelet-decomposed signal into 3 sub-bands (Variance, Mean energy, Minimum energy, Maximum energy, Mean, Minimum amplitude, Standard deviation (STD), Maximum amplitude, Range, Peak-to-peak, Root‑mean‑square (RMS), Peak magnitude to RMS ratio) from each detail (D1–D3) and approximation (A1–A3) |
| PPG | 486 | 486.0.1 / Exp 2 Bior4.4 D1 | PPG | handcrafted | low | no | time-domain statistical features from wavelet-decomposed signal into 3 sub-bands (Variance, Mean energy, Minimum energy, Maximum energy, Mean, Minimum amplitude, Standard deviation (STD), Maximum amplitude, Range, Peak-to-peak, Root‑mean‑square (RMS), Peak magnitude to RMS ratio) from each detail (D1–D3) and approximation (A1–A3) |
| PPG | 486 | 486.0.1 / Exp 2 Bior4.4 D2 | PPG | handcrafted | low | no | time-domain statistical features from wavelet-decomposed signal into 3 sub-bands (Variance, Mean energy, Minimum energy, Maximum energy, Mean, Minimum amplitude, Standard deviation (STD), Maximum amplitude, Range, Peak-to-peak, Root‑mean‑square (RMS), Peak magnitude to RMS ratio) from each detail (D1–D3) and approximation (A1–A3) |
| PPG | 486 | 486.0.1 / Exp 2 Bior4.4 D3 | PPG | handcrafted | low | no | time-domain statistical features from wavelet-decomposed signal into 3 sub-bands (Variance, Mean energy, Minimum energy, Maximum energy, Mean, Minimum amplitude, Standard deviation (STD), Maximum amplitude, Range, Peak-to-peak, Root‑mean‑square (RMS), Peak magnitude to RMS ratio) from each detail (D1–D3) and approximation (A1–A3) |
| PPG | 486 | 486.0.1 / Exp 2 Bior5.5 A1 | PPG | handcrafted | low | no | time-domain statistical features from wavelet-decomposed signal into 3 sub-bands (Variance, Mean energy, Minimum energy, Maximum energy, Mean, Minimum amplitude, Standard deviation (STD), Maximum amplitude, Range, Peak-to-peak, Root‑mean‑square (RMS), Peak magnitude to RMS ratio) from each detail (D1–D3) and approximation (A1–A3) |
| PPG | 486 | 486.0.1 / Exp 2 Bior5.5 A2 | PPG | handcrafted | low | no | time-domain statistical features from wavelet-decomposed signal into 3 sub-bands (Variance, Mean energy, Minimum energy, Maximum energy, Mean, Minimum amplitude, Standard deviation (STD), Maximum amplitude, Range, Peak-to-peak, Root‑mean‑square (RMS), Peak magnitude to RMS ratio) from each detail (D1–D3) and approximation (A1–A3) |
| PPG | 486 | 486.0.1 / Exp 2 Bior5.5 A3 | PPG | handcrafted | low | no | time-domain statistical features from wavelet-decomposed signal into 3 sub-bands (Variance, Mean energy, Minimum energy, Maximum energy, Mean, Minimum amplitude, Standard deviation (STD), Maximum amplitude, Range, Peak-to-peak, Root‑mean‑square (RMS), Peak magnitude to RMS ratio) from each detail (D1–D3) and approximation (A1–A3) |
| PPG | 486 | 486.0.1 / Exp 2 Bior5.5 D1 | PPG | handcrafted | low | no | time-domain statistical features from wavelet-decomposed signal into 3 sub-bands (Variance, Mean energy, Minimum energy, Maximum energy, Mean, Minimum amplitude, Standard deviation (STD), Maximum amplitude, Range, Peak-to-peak, Root‑mean‑square (RMS), Peak magnitude to RMS ratio) from each detail (D1–D3) and approximation (A1–A3) |
| PPG | 486 | 486.0.1 / Exp 2 Bior5.5 D2 | PPG | handcrafted | low | no | time-domain statistical features from wavelet-decomposed signal into 3 sub-bands (Variance, Mean energy, Minimum energy, Maximum energy, Mean, Minimum amplitude, Standard deviation (STD), Maximum amplitude, Range, Peak-to-peak, Root‑mean‑square (RMS), Peak magnitude to RMS ratio) from each detail (D1–D3) and approximation (A1–A3) |
| PPG | 486 | 486.0.1 / Exp 2 Bior5.5 D3 | PPG | handcrafted | low | no | time-domain statistical features from wavelet-decomposed signal into 3 sub-bands (Variance, Mean energy, Minimum energy, Maximum energy, Mean, Minimum amplitude, Standard deviation (STD), Maximum amplitude, Range, Peak-to-peak, Root‑mean‑square (RMS), Peak magnitude to RMS ratio) from each detail (D1–D3) and approximation (A1–A3) |
| PPG | 486 | 486.0.1 / Exp 2 Bior6.8 A1 | PPG | handcrafted | low | no | time-domain statistical features from wavelet-decomposed signal into 3 sub-bands (Variance, Mean energy, Minimum energy, Maximum energy, Mean, Minimum amplitude, Standard deviation (STD), Maximum amplitude, Range, Peak-to-peak, Root‑mean‑square (RMS), Peak magnitude to RMS ratio) from each detail (D1–D3) and approximation (A1–A3) |
| PPG | 486 | 486.0.1 / Exp 2 Bior6.8 A2 | PPG | handcrafted | low | no | time-domain statistical features from wavelet-decomposed signal into 3 sub-bands (Variance, Mean energy, Minimum energy, Maximum energy, Mean, Minimum amplitude, Standard deviation (STD), Maximum amplitude, Range, Peak-to-peak, Root‑mean‑square (RMS), Peak magnitude to RMS ratio) from each detail (D1–D3) and approximation (A1–A3) |
| PPG | 486 | 486.0.1 / Exp 2 Bior6.8 A3 | PPG | handcrafted | low | no | time-domain statistical features from wavelet-decomposed signal into 3 sub-bands (Variance, Mean energy, Minimum energy, Maximum energy, Mean, Minimum amplitude, Standard deviation (STD), Maximum amplitude, Range, Peak-to-peak, Root‑mean‑square (RMS), Peak magnitude to RMS ratio) from each detail (D1–D3) and approximation (A1–A3) |
| PPG | 486 | 486.0.1 / Exp 2 Bior6.8 D1 | PPG | handcrafted | low | no | time-domain statistical features from wavelet-decomposed signal into 3 sub-bands (Variance, Mean energy, Minimum energy, Maximum energy, Mean, Minimum amplitude, Standard deviation (STD), Maximum amplitude, Range, Peak-to-peak, Root‑mean‑square (RMS), Peak magnitude to RMS ratio) from each detail (D1–D3) and approximation (A1–A3) |
| PPG | 486 | 486.0.1 / Exp 2 Bior6.8 D2 | PPG | handcrafted | low | no | time-domain statistical features from wavelet-decomposed signal into 3 sub-bands (Variance, Mean energy, Minimum energy, Maximum energy, Mean, Minimum amplitude, Standard deviation (STD), Maximum amplitude, Range, Peak-to-peak, Root‑mean‑square (RMS), Peak magnitude to RMS ratio) from each detail (D1–D3) and approximation (A1–A3) |
| PPG | 486 | 486.0.1 / Exp 2 Bior6.8 D3 | PPG | handcrafted | low | no | time-domain statistical features from wavelet-decomposed signal into 3 sub-bands (Variance, Mean energy, Minimum energy, Maximum energy, Mean, Minimum amplitude, Standard deviation (STD), Maximum amplitude, Range, Peak-to-peak, Root‑mean‑square (RMS), Peak magnitude to RMS ratio) from each detail (D1–D3) and approximation (A1–A3) |
| PPG | 486 | 486.0.1 / Exp 3 Bior1.1 Approx. Coefficient Fusion | PPG | handcrafted | medium | no | time-domain statistical features from wavelet-decomposed signal into 3 sub-bands (Variance, Mean energy, Minimum energy, Maximum energy, Mean, Minimum amplitude, Standard deviation (STD), Maximum amplitude, Range, Peak-to-peak, Root‑mean‑square (RMS), Peak magnitude to RMS ratio) from Approximation of Coefficient Fusion Bior 1.1 |
| PPG | 486 | 486.0.1 / Exp 3 Bior1.1 Coefficient Fusion | PPG | handcrafted | medium | no | time-domain statistical features from wavelet-decomposed signal into 3 sub-bands (Variance, Mean energy, Minimum energy, Maximum energy, Mean, Minimum amplitude, Standard deviation (STD), Maximum amplitude, Range, Peak-to-peak, Root‑mean‑square (RMS), Peak magnitude to RMS ratio) from Detail Coefficient Fusion Bior 1.1 |
| PPG | 486 | 486.0.1 / Exp 3 Bior1.3 Approx. Coefficient Fusion | PPG | handcrafted | medium | no | time-domain statistical features from wavelet-decomposed signal into 3 sub-bands (Variance, Mean energy, Minimum energy, Maximum energy, Mean, Minimum amplitude, Standard deviation (STD), Maximum amplitude, Range, Peak-to-peak, Root‑mean‑square (RMS), Peak magnitude to RMS ratio) from Approximation of Coefficient Fusion Bior 1.3 |
| PPG | 486 | 486.0.1 / Exp 3 Bior1.3 Coefficient Fusion | PPG | handcrafted | medium | no | time-domain statistical features from wavelet-decomposed signal into 3 sub-bands (Variance, Mean energy, Minimum energy, Maximum energy, Mean, Minimum amplitude, Standard deviation (STD), Maximum amplitude, Range, Peak-to-peak, Root‑mean‑square (RMS), Peak magnitude to RMS ratio) from Detail Coefficient Fusion Bior 1.3 |
| PPG | 486 | Approx | PPG | handcrafted | medium | no | time-domain statistical features from wavelet-decomposed signal into 3 sub-bands (Variance, Mean energy, Minimum energy, Maximum energy, Mean, Minimum amplitude, Standard deviation (STD), Maximum amplitude, Range, Peak-to-peak, Root‑mean‑square (RMS), Peak magnitude to RMS ratio) from Approximation of Coefficient Fusion |
| PPG | 486 | Coef | PPG | handcrafted | medium | no | time-domain statistical features from wavelet-decomposed signal into 3 sub-bands (Variance, Mean energy, Minimum energy, Maximum energy, Mean, Minimum amplitude, Standard deviation (STD), Maximum amplitude, Range, Peak-to-peak, Root‑mean‑square (RMS), Peak magnitude to RMS ratio) from Detail Coefficient Fusion |
| PPG | 55 | Empatica LR 7/3 | PPG | handcrafted | low | no | Mean RR, STD RR, RMSSD, pNN50, HRV triangular index, TINN, SDSD, LF, HF, LF/HF, VLF |
| PPG | 55 | Empatica LR 9/1 | PPG | handcrafted | low | no | Mean RR, STD RR, RMSSD, pNN50, HRV triangular index, TINN, SDSD, LF, HF, LF/HF, VLF |
| PPG | 55 | Empatica MLP 7/3 | PPG | handcrafted | low | no | Mean RR, STD RR, RMSSD, pNN50, HRV triangular index, TINN, SDSD, LF, HF, LF/HF, VLF |
| PPG | 55 | Empatica MLP 9/1 | PPG | handcrafted | low | no | Mean RR, STD RR, RMSSD, pNN50, HRV triangular index, TINN, SDSD, LF, HF, LF/HF, VLF |
| PPG | 55 | Empatica NB 7/3 | PPG | handcrafted | low | no | Mean RR, STD RR, RMSSD, pNN50, HRV triangular index, TINN, SDSD, LF, HF, LF/HF, VLF |
| PPG | 55 | Empatica NB 9/1 | PPG | handcrafted | low | no | Mean RR, STD RR, RMSSD, pNN50, HRV triangular index, TINN, SDSD, LF, HF, LF/HF, VLF |
| PPG | 55 | Empatica RF 7/3 | PPG | handcrafted | low | no | Mean RR, STD RR, RMSSD, pNN50, HRV triangular index, TINN, SDSD, LF, HF, LF/HF, VLF |
| PPG | 55 | Empatica RF 9/1 | PPG | handcrafted | low | no | Mean RR, STD RR, RMSSD, pNN50, HRV triangular index, TINN, SDSD, LF, HF, LF/HF, VLF |
| PPG | 55 | Empatica kNN 7/3 | PPG | handcrafted | low | no | Mean RR, STD RR, RMSSD, pNN50, HRV triangular index, TINN, SDSD, LF, HF, LF/HF, VLF |
| PPG | 55 | Empatica kNN 9/1 | PPG | handcrafted | low | no | Mean RR, STD RR, RMSSD, pNN50, HRV triangular index, TINN, SDSD, LF, HF, LF/HF, VLF |
| PPG | 55 | S LR 7/3 | PPG | handcrafted | low | no | Mean RR, STD RR, RMSSD, pNN50, HRV triangular index, TINN, SDSD, LF, HF, LF/HF, VLF |
| PPG | 55 | S LR 9/1 | PPG | handcrafted | low | no | Mean RR, STD RR, RMSSD, pNN50, HRV triangular index, TINN, SDSD, LF, HF, LF/HF, VLF |
| PPG | 55 | S MLP 7/3 | PPG | handcrafted | low | no | Mean RR, STD RR, RMSSD, pNN50, HRV triangular index, TINN, SDSD, LF, HF, LF/HF, VLF |
| PPG | 55 | S MLP 9/1 | PPG | handcrafted | low | no | Mean RR, STD RR, RMSSD, pNN50, HRV triangular index, TINN, SDSD, LF, HF, LF/HF, VLF |
| PPG | 55 | S NB 7/3 | PPG | handcrafted | low | no | Mean RR, STD RR, RMSSD, pNN50, HRV triangular index, TINN, SDSD, LF, HF, LF/HF, VLF |
| PPG | 55 | S NB 9/1 | PPG | handcrafted | low | no | Mean RR, STD RR, RMSSD, pNN50, HRV triangular index, TINN, SDSD, LF, HF, LF/HF, VLF |
| PPG | 55 | S RF 7/3 | PPG | handcrafted | low | no | Mean RR, STD RR, RMSSD, pNN50, HRV triangular index, TINN, SDSD, LF, HF, LF/HF, VLF |
| PPG | 55 | S RF 9/1 | PPG | handcrafted | low | no | Mean RR, STD RR, RMSSD, pNN50, HRV triangular index, TINN, SDSD, LF, HF, LF/HF, VLF |
| PPG | 55 | S kNN 7/3 | PPG | handcrafted | low | no | Mean RR, STD RR, RMSSD, pNN50, HRV triangular index, TINN, SDSD, LF, HF, LF/HF, VLF |
| PPG | 55 | S kNN 9/1 | PPG | handcrafted | low | no | Mean RR, STD RR, RMSSD, pNN50, HRV triangular index, TINN, SDSD, LF, HF, LF/HF, VLF |
| PPG | 55 | S2 LR 7/3 | PPG | handcrafted | low | no | Mean RR, STD RR, RMSSD, pNN50, HRV triangular index, TINN, SDSD, LF, HF, LF/HF, VLF |
| PPG | 55 | S2 LR 9/1 | PPG | handcrafted | low | no | Mean RR, STD RR, RMSSD, pNN50, HRV triangular index, TINN, SDSD, LF, HF, LF/HF, VLF |
| PPG | 55 | S2 MLP 7/3 | PPG | handcrafted | low | no | Mean RR, STD RR, RMSSD, pNN50, HRV triangular index, TINN, SDSD, LF, HF, LF/HF, VLF |
| PPG | 55 | S2 MLP 9/1 | PPG | handcrafted | low | no | Mean RR, STD RR, RMSSD, pNN50, HRV triangular index, TINN, SDSD, LF, HF, LF/HF, VLF |
| PPG | 55 | S2 NB 7/3 | PPG | handcrafted | low | no | Mean RR, STD RR, RMSSD, pNN50, HRV triangular index, TINN, SDSD, LF, HF, LF/HF, VLF |
| PPG | 55 | S2 NB 9/1 | PPG | handcrafted | low | no | Mean RR, STD RR, RMSSD, pNN50, HRV triangular index, TINN, SDSD, LF, HF, LF/HF, VLF |
| PPG | 55 | S2 RF 7/3 | PPG | handcrafted | low | no | Mean RR, STD RR, RMSSD, pNN50, HRV triangular index, TINN, SDSD, LF, HF, LF/HF, VLF |
| PPG | 55 | S2 RF 9/1 | PPG | handcrafted | low | no | Mean RR, STD RR, RMSSD, pNN50, HRV triangular index, TINN, SDSD, LF, HF, LF/HF, VLF |
| PPG | 55 | S2 kNN 7/3 | PPG | handcrafted | low | no | Mean RR, STD RR, RMSSD, pNN50, HRV triangular index, TINN, SDSD, LF, HF, LF/HF, VLF |
| PPG | 55 | S2 kNN 9/1 | PPG | handcrafted | low | no | Mean RR, STD RR, RMSSD, pNN50, HRV triangular index, TINN, SDSD, LF, HF, LF/HF, VLF |
| PPG | 61 | L1HR binary SVM | PPG | handcrafted | low | yes | 21 statistical features: mean, median, standard deviation, variance, coef cient of variance, range, coefficient of range, first quartile or 25th percentile, third quartile or 75th percentile, maximum, interquartile range, coefficient of interquartile, mean absolute deviation, median absolute deviation, energy, power, root mean square, root sum of squares, signal to noise ratio, skewness, and kurtosis |
| PPG | 61 | L1HR unary SVM | PPG | handcrafted | low | no | 21 statistical features: mean, median, standard deviation, variance, coef cient of variance, range, coefficient of range, first quartile or 25th percentile, third quartile or 75th percentile, maximum, interquartile range, coefficient of interquartile, mean absolute deviation, median absolute deviation, energy, power, root mean square, root sum of squares, signal to noise ratio, skewness, and kurtosis |
| PPG | 61 | L3aHR+Resp binary SVM | PPG+microphone | handcrafted | low | yes | HRV: 21 statistical features: mean, median, standard deviation, variance, coef cient of variance, range, coefficient of range, first quartile or 25th percentile, third quartile or 75th percentile, maximum, interquartile range, coefficient of interquartile, mean absolute deviation, median absolute deviation, energy, power, root mean square, root sum of squares, signal to noise ratio, skewness, and kurtosis; Resp: 182 features, 40 Mel-frequency cepstral coefficients (MFCCs), 13 Gammatone frequency cepstral coef cients (GFCCs), 13 Bark frequency cepstral coef cients (BFCCs), 13 Linear predictive coding (LPC), 13 Revised perceptual linear prediction coef cients (RPLP), 12 chromas, and six tonal features. 5 statistical features from each chroma feature and tonal feature. |
| PPG | 61 | L3aHR+Resp unary SVM | PPG+microphone | handcrafted | medium | no | HRV: 21 statistical features: mean, median, standard deviation, variance, coef cient of variance, range, coefficient of range, first quartile or 25th percentile, third quartile or 75th percentile, maximum, interquartile range, coefficient of interquartile, mean absolute deviation, median absolute deviation, energy, power, root mean square, root sum of squares, signal to noise ratio, skewness, and kurtosis; Resp: 182 features, 40 Mel-frequency cepstral coefficients (MFCCs), 13 Gammatone frequency cepstral coef cients (GFCCs), 13 Bark frequency cepstral coef cients (BFCCs), 13 Linear predictive coding (LPC), 13 Revised perceptual linear prediction coef cients (RPLP), 12 chromas, and six tonal features. 5 statistical features from each chroma feature and tonal feature. |
| Other | 147 | Heart Dorsal-Ventral Post-Exercise | SCG | handcrafted | not coded | no | cleaned time series |
| Other | 147 | Heart Dorsal-Ventral Resilient Post-Exercise | SCG | handcrafted | not coded | no | cleaned time series |
| Other | 147 | Heart Dorsal-Ventral Resilient Sit | SCG | handcrafted | not coded | no | cleaned time series |
| Other | 147 | Heart Dorsal-Ventral Resilient Stand | SCG | handcrafted | not coded | no | cleaned time series |
| Other | 147 | Heart Dorsal-Ventral Resilient Walk | SCG | handcrafted | not coded | no | cleaned time series |
| Other | 147 | Heart Dorsal-Ventral Sit | SCG | handcrafted | not coded | no | cleaned time series |
| Other | 147 | Heart Dorsal-Ventral Stand | SCG | handcrafted | not coded | no | cleaned time series |
| Other | 147 | Heart Dorsal-Ventral Walk | SCG | handcrafted | not coded | no | cleaned time series |
| Other | 147 | Heart Horizontal Post-Exercise | SCG | handcrafted | not coded | no | cleaned time series |
| Other | 147 | Heart Horizontal Resilient Post-Exercise | SCG | handcrafted | not coded | no | cleaned time series |
| Other | 147 | Heart Horizontal Resilient Sit | SCG | handcrafted | not coded | no | cleaned time series |
| Other | 147 | Heart Horizontal Resilient Stand | SCG | handcrafted | not coded | no | cleaned time series |
| Other | 147 | Heart Horizontal Resilient Walk | SCG | handcrafted | not coded | no | cleaned time series |
| Other | 147 | Heart Horizontal Sit | SCG | handcrafted | not coded | no | cleaned time series |
| Other | 147 | Heart Horizontal Stand | SCG | handcrafted | not coded | no | cleaned time series |
| Other | 147 | Heart Horizontal Walk | SCG | handcrafted | not coded | no | cleaned time series |
| Other | 147 | Heart Vertical Post-Exercise | SCG | handcrafted | not coded | no | cleaned time series |
| Other | 147 | Heart Vertical Resilient Post-Exercise | SCG | handcrafted | not coded | no | cleaned time series |
| Other | 147 | Heart Vertical Resilient Sit | SCG | handcrafted | not coded | no | cleaned time series |
| Other | 147 | Heart Vertical Resilient Stand | SCG | handcrafted | not coded | no | cleaned time series |
| Other | 147 | Heart Vertical Resilient Walk | SCG | handcrafted | not coded | no | cleaned time series |
| Other | 147 | Heart Vertical Sit | SCG | handcrafted | not coded | no | cleaned time series |
| Other | 147 | Heart Vertical Stand | SCG | handcrafted | not coded | no | cleaned time series |
| Other | 147 | Heart Vertical Walk | SCG | handcrafted | not coded | no | cleaned time series |
| Other | 147 | Left Carotid Dorsal-Ventral Post-Exercise | SCG | handcrafted | not coded | no | cleaned time series |
| Other | 147 | Left Carotid Dorsal-Ventral Resilient Post-Exercise | SCG | handcrafted | not coded | no | cleaned time series |
| Other | 147 | Left Carotid Dorsal-Ventral Resilient Sit | SCG | handcrafted | not coded | no | cleaned time series |
| Other | 147 | Left Carotid Dorsal-Ventral Resilient Stand | SCG | handcrafted | not coded | no | cleaned time series |
| Other | 147 | Left Carotid Dorsal-Ventral Resilient Walk | SCG | handcrafted | not coded | no | cleaned time series |
| Other | 147 | Left Carotid Dorsal-Ventral Sit | SCG | handcrafted | not coded | no | cleaned time series |
| Other | 147 | Left Carotid Dorsal-Ventral Stand | SCG | handcrafted | not coded | no | cleaned time series |
| Other | 147 | Left Carotid Dorsal-Ventral Walk | SCG | handcrafted | not coded | no | cleaned time series |
| Other | 147 | Left Carotid Horizontal Post-Exercise | SCG | handcrafted | not coded | no | cleaned time series |
| Other | 147 | Left Carotid Horizontal Resilient Post-Exercise | SCG | handcrafted | not coded | no | cleaned time series |
| Other | 147 | Left Carotid Horizontal Resilient Sit | SCG | handcrafted | not coded | no | cleaned time series |
| Other | 147 | Left Carotid Horizontal Resilient Stand | SCG | handcrafted | not coded | no | cleaned time series |
| Other | 147 | Left Carotid Horizontal Resilient Walk | SCG | handcrafted | not coded | no | cleaned time series |
| Other | 147 | Left Carotid Horizontal Sit | SCG | handcrafted | not coded | no | cleaned time series |
| Other | 147 | Left Carotid Horizontal Stand | SCG | handcrafted | not coded | no | cleaned time series |
| Other | 147 | Left Carotid Horizontal Walk | SCG | handcrafted | not coded | no | cleaned time series |
| Other | 147 | Left Carotid Vertical Post-Exercise | SCG | handcrafted | not coded | no | cleaned time series |
| Other | 147 | Left Carotid Vertical Resilient Post-Exercise | SCG | handcrafted | not coded | no | cleaned time series |
| Other | 147 | Left Carotid Vertical Resilient Sit | SCG | handcrafted | not coded | no | cleaned time series |
| Other | 147 | Left Carotid Vertical Resilient Stand | SCG | handcrafted | not coded | no | cleaned time series |
| Other | 147 | Left Carotid Vertical Resilient Walk | SCG | handcrafted | not coded | no | cleaned time series |
| Other | 147 | Left Carotid Vertical Sit | SCG | handcrafted | not coded | no | cleaned time series |
| Other | 147 | Left Carotid Vertical Stand | SCG | handcrafted | not coded | no | cleaned time series |
| Other | 147 | Left Carotid Vertical Walk | SCG | handcrafted | not coded | no | cleaned time series |
| Other | 147 | Left Hand Dorsal-Ventral Post-Exercise | SCG | handcrafted | not coded | no | cleaned time series |
| Other | 147 | Left Hand Dorsal-Ventral Resilient Post-Exercise | SCG | handcrafted | not coded | no | cleaned time series |
| Other | 147 | Left Hand Dorsal-Ventral Resilient Sit | SCG | handcrafted | not coded | no | cleaned time series |
| Other | 147 | Left Hand Dorsal-Ventral Resilient Stand | SCG | handcrafted | not coded | no | cleaned time series |
| Other | 147 | Left Hand Dorsal-Ventral Resilient Walk | SCG | handcrafted | not coded | no | cleaned time series |
| Other | 147 | Left Hand Dorsal-Ventral Sit | SCG | handcrafted | not coded | no | cleaned time series |
| Other | 147 | Left Hand Dorsal-Ventral Stand | SCG | handcrafted | not coded | no | cleaned time series |
| Other | 147 | Left Hand Dorsal-Ventral Walk | SCG | handcrafted | not coded | no | cleaned time series |
| Other | 147 | Left Hand Horizontal Post-Exercise | SCG | handcrafted | not coded | no | cleaned time series |
| Other | 147 | Left Hand Horizontal Resilient Post-Exercise | SCG | handcrafted | not coded | no | cleaned time series |
| Other | 147 | Left Hand Horizontal Resilient Sit | SCG | handcrafted | not coded | no | cleaned time series |
| Other | 147 | Left Hand Horizontal Resilient Stand | SCG | handcrafted | not coded | no | cleaned time series |
| Other | 147 | Left Hand Horizontal Resilient Walk | SCG | handcrafted | not coded | no | cleaned time series |
| Other | 147 | Left Hand Horizontal Sit | SCG | handcrafted | not coded | no | cleaned time series |
| Other | 147 | Left Hand Horizontal Stand | SCG | handcrafted | not coded | no | cleaned time series |
| Other | 147 | Left Hand Horizontal Walk | SCG | handcrafted | not coded | no | cleaned time series |
| Other | 147 | Left Hand Vertical Post-Exercise | SCG | handcrafted | not coded | no | cleaned time series |
| Other | 147 | Left Hand Vertical Resilient Post-Exercise | SCG | handcrafted | not coded | no | cleaned time series |
| Other | 147 | Left Hand Vertical Resilient Sit | SCG | handcrafted | not coded | no | cleaned time series |
| Other | 147 | Left Hand Vertical Resilient Stand | SCG | handcrafted | not coded | no | cleaned time series |
| Other | 147 | Left Hand Vertical Resilient Walk | SCG | handcrafted | not coded | no | cleaned time series |
| Other | 147 | Left Hand Vertical Sit | SCG | handcrafted | not coded | no | cleaned time series |
| Other | 147 | Left Hand Vertical Stand | SCG | handcrafted | not coded | no | cleaned time series |
| Other | 147 | Left Hand Vertical Walk | SCG | handcrafted | not coded | no | cleaned time series |
| Other | 147 | Right Hand Dorsal-Ventral Post-Exercise | SCG | handcrafted | not coded | no | cleaned time series |
| Other | 147 | Right Hand Dorsal-Ventral Resilient Post-Exercise | SCG | handcrafted | not coded | no | cleaned time series |
| Other | 147 | Right Hand Dorsal-Ventral Resilient Sit | SCG | handcrafted | not coded | no | cleaned time series |
| Other | 147 | Right Hand Dorsal-Ventral Resilient Stand | SCG | handcrafted | not coded | no | cleaned time series |
| Other | 147 | Right Hand Dorsal-Ventral Resilient Walk | SCG | handcrafted | not coded | no | cleaned time series |
| Other | 147 | Right Hand Dorsal-Ventral Sit | SCG | handcrafted | not coded | no | cleaned time series |
| Other | 147 | Right Hand Dorsal-Ventral Stand | SCG | handcrafted | not coded | no | cleaned time series |
| Other | 147 | Right Hand Dorsal-Ventral Walk | SCG | handcrafted | not coded | no | cleaned time series |
| Other | 147 | Right Hand Horizontal Post-Exercise | SCG | handcrafted | not coded | no | cleaned time series |
| Other | 147 | Right Hand Horizontal Resilient Post-Exercise | SCG | handcrafted | not coded | no | cleaned time series |
| Other | 147 | Right Hand Horizontal Resilient Sit | SCG | handcrafted | not coded | no | cleaned time series |
| Other | 147 | Right Hand Horizontal Resilient Stand | SCG | handcrafted | not coded | no | cleaned time series |
| Other | 147 | Right Hand Horizontal Resilient Walk | SCG | handcrafted | not coded | no | cleaned time series |
| Other | 147 | Right Hand Horizontal Sit | SCG | handcrafted | not coded | no | cleaned time series |
| Other | 147 | Right Hand Horizontal Stand | SCG | handcrafted | not coded | no | cleaned time series |
| Other | 147 | Right Hand Horizontal Walk | SCG | handcrafted | not coded | no | cleaned time series |
| Other | 147 | Right Hand Vertical Post-Exercise | SCG | handcrafted | not coded | no | cleaned time series |
| Other | 147 | Right Hand Vertical Resilient Post-Exercise | SCG | handcrafted | not coded | no | cleaned time series |
| Other | 147 | Right Hand Vertical Resilient Sit | SCG | handcrafted | not coded | no | cleaned time series |
| Other | 147 | Right Hand Vertical Resilient Stand | SCG | handcrafted | not coded | no | cleaned time series |
| Other | 147 | Right Hand Vertical Resilient Walk | SCG | handcrafted | not coded | no | cleaned time series |
| Other | 147 | Right Hand Vertical Sit | SCG | handcrafted | not coded | no | cleaned time series |
| Other | 147 | Right Hand Vertical Stand | SCG | handcrafted | not coded | no | cleaned time series |
| Other | 147 | Right Hand Vertical Walk | SCG | handcrafted | not coded | no | cleaned time series |
| Other | 147 | Sternum Dorsal-Ventral Post-Exercise | SCG | handcrafted | not coded | no | cleaned time series |
| Other | 147 | Sternum Dorsal-Ventral Resilient Post-Exercise | SCG | handcrafted | not coded | no | cleaned time series |
| Other | 147 | Sternum Dorsal-Ventral Resilient Sit | SCG | handcrafted | not coded | no | cleaned time series |
| Other | 147 | Sternum Dorsal-Ventral Resilient Stand | SCG | handcrafted | not coded | no | cleaned time series |
| Other | 147 | Sternum Dorsal-Ventral Resilient Walk | SCG | handcrafted | not coded | no | cleaned time series |
| Other | 147 | Sternum Dorsal-Ventral Sit | SCG | handcrafted | not coded | no | cleaned time series |
| Other | 147 | Sternum Dorsal-Ventral Stand | SCG | handcrafted | not coded | no | cleaned time series |
| Other | 147 | Sternum Dorsal-Ventral Walk | SCG | handcrafted | not coded | no | cleaned time series |
| Other | 147 | Sternum Horizontal Post-Exercise | SCG | handcrafted | not coded | no | cleaned time series |
| Other | 147 | Sternum Horizontal Resilient Post-Exercise | SCG | handcrafted | not coded | no | cleaned time series |
| Other | 147 | Sternum Horizontal Resilient Sit | SCG | handcrafted | not coded | no | cleaned time series |
| Other | 147 | Sternum Horizontal Resilient Stand | SCG | handcrafted | not coded | no | cleaned time series |
| Other | 147 | Sternum Horizontal Resilient Walk | SCG | handcrafted | not coded | no | cleaned time series |
| Other | 147 | Sternum Horizontal Sit | SCG | handcrafted | not coded | no | cleaned time series |
| Other | 147 | Sternum Horizontal Stand | SCG | handcrafted | not coded | no | cleaned time series |
| Other | 147 | Sternum Horizontal Walk | SCG | handcrafted | not coded | no | cleaned time series |
| Other | 147 | Sternum Vertical Post-Exercise | SCG | handcrafted | not coded | no | cleaned time series |
| Other | 147 | Sternum Vertical Resilient Post-Exercise | SCG | handcrafted | not coded | no | cleaned time series |
| Other | 147 | Sternum Vertical Resilient Sit | SCG | handcrafted | not coded | no | cleaned time series |
| Other | 147 | Sternum Vertical Resilient Stand | SCG | handcrafted | not coded | no | cleaned time series |
| Other | 147 | Sternum Vertical Resilient Walk | SCG | handcrafted | not coded | no | cleaned time series |
| Other | 147 | Sternum Vertical Sit | SCG | handcrafted | not coded | no | cleaned time series |
| Other | 147 | Sternum Vertical Stand | SCG | handcrafted | not coded | no | cleaned time series |
| Other | 147 | Sternum Vertical Walk | SCG | handcrafted | not coded | no | cleaned time series |
| Other | 189 | NN-FF 2 | EDA | handcrafted | low | no | 12 statistical features (mean, maximum amplitude, minimum amplitude, maximum energy, minimum energy, standard deviation, peak to peak, median, root-mean-square (RMS) and peakmagnitude-to-RMS ratio) on discreet wavelet transform (DWT) coefficient and 6 entropy features (Shannon entropy, energy entropy, threshold entropy, sure entropy and normalised entropy) to wavelet packet. |
| Other | 189 | NN-FF 3 | EDA | handcrafted | low | no | 12 statistical features (mean, maximum amplitude, minimum amplitude, maximum energy, minimum energy, standard deviation, peak to peak, median, root-mean-square (RMS) and peakmagnitude-to-RMS ratio) on discreet wavelet transform (DWT) coefficient and 6 entropy features (Shannon entropy, energy entropy, threshold entropy, sure entropy and normalised entropy) to wavelet packet. |
| Other | 189 | NN-FF best score | EDA | handcrafted | low | no | 12 statistical features (mean, maximum amplitude, minimum amplitude, maximum energy, minimum energy, standard deviation, peak to peak, median, root-mean-square (RMS) and peakmagnitude-to-RMS ratio) on discreet wavelet transform (DWT) coefficient and 6 entropy features (Shannon entropy, energy entropy, threshold entropy, sure entropy and normalised entropy) to wavelet packet. |
| Other | 189 | NN-FF intraday | EDA | handcrafted | low | no | 12 statistical features (mean, maximum amplitude, minimum amplitude, maximum energy, minimum energy, standard deviation, peak to peak, median, root-mean-square (RMS) and peakmagnitude-to-RMS ratio) on discreet wavelet transform (DWT) coefficient and 6 entropy features (Shannon entropy, energy entropy, threshold entropy, sure entropy and normalised entropy) to wavelet packet. |
| Other | 305 | LR 120 EDA | EDA | handcrafted | low | no | EDA features: Mean Tonic Component, SD Tonic (Standard deviation of the tonic component), Perc20 (20th percentile of the phasic component), Perc80 Tonic (80th percentile of the tonic component), Quartdev Tonic (Quartile deviation 75 percentile - 25 percentile of the tonic component), Strong Peaks (The number of strong peaks per 100 seconds) and Peaks Phasic (The number of peaks per 100 seconds). |
| Other | 305 | LR 120 EDA-HRV | EDA, PPG | handcrafted | low | no | EDA features: Mean Tonic Component, SD Tonic (Standard deviation of the tonic component), Perc20 (20th percentile of the phasic component), Perc80 Tonic (80th percentile of the tonic component), Quartdev Tonic (Quartile deviation 75 percentile - 25 percentile of the tonic component), Strong Peaks (The number of strong peaks per 100 seconds) and Peaks Phasic (The number of peaks per 100 seconds). HRV features: Time Domain Features: Mean HR, standard deviation of the inter-beat interval (STD RR), mean value of the inter-beat (RR) intervals (Mean RR), root mean square of successive difference of the RR intervals (RMSSD), the percentage of the number of successive RR intervals varying more than 50ms from the previous interval (pNN50), the total number of RR intervals divided by the height of the histogram of all RR intervals measured on a scale with bins of 1/128 s (HRV triangular index), and triangular interpolation of RR interval histogram (TINN). Frequency Domain Features: Low frequency power (LF), high-frequency power (HF), very low frequency power (VLF), prevalent low frequency (pLF), prevalent high frequency (pHF), the ratio of LF to HF (LF/HF) (Preprocessing +FFT), LF, HF, LF/HF (Lomb-Scargle) |
| Other | 305 | LR 30 EDA | EDA | handcrafted | low | no | EDA features: Mean Tonic Component, SD Tonic (Standard deviation of the tonic component), Perc20 (20th percentile of the phasic component), Perc80 Tonic (80th percentile of the tonic component), Quartdev Tonic (Quartile deviation 75 percentile - 25 percentile of the tonic component), Strong Peaks (The number of strong peaks per 100 seconds) and Peaks Phasic (The number of peaks per 100 seconds). |
| Other | 305 | LR 30 EDA-HRV | EDA, PPG | handcrafted | low | no | EDA features: Mean Tonic Component, SD Tonic (Standard deviation of the tonic component), Perc20 (20th percentile of the phasic component), Perc80 Tonic (80th percentile of the tonic component), Quartdev Tonic (Quartile deviation 75 percentile - 25 percentile of the tonic component), Strong Peaks (The number of strong peaks per 100 seconds) and Peaks Phasic (The number of peaks per 100 seconds). HRV features: Time Domain Features: Mean HR, standard deviation of the inter-beat interval (STD RR), mean value of the inter-beat (RR) intervals (Mean RR), root mean square of successive difference of the RR intervals (RMSSD), the percentage of the number of successive RR intervals varying more than 50ms from the previous interval (pNN50), the total number of RR intervals divided by the height of the histogram of all RR intervals measured on a scale with bins of 1/128 s (HRV triangular index), and triangular interpolation of RR interval histogram (TINN). Frequency Domain Features: Low frequency power (LF), high-frequency power (HF), very low frequency power (VLF), prevalent low frequency (pLF), prevalent high frequency (pHF), the ratio of LF to HF (LF/HF) (Preprocessing +FFT), LF, HF, LF/HF (Lomb-Scargle) |
| Other | 305 | LR 60 EDA | EDA | handcrafted | low | no | EDA features: Mean Tonic Component, SD Tonic (Standard deviation of the tonic component), Perc20 (20th percentile of the phasic component), Perc80 Tonic (80th percentile of the tonic component), Quartdev Tonic (Quartile deviation 75 percentile - 25 percentile of the tonic component), Strong Peaks (The number of strong peaks per 100 seconds) and Peaks Phasic (The number of peaks per 100 seconds). |
| Other | 305 | LR 60 EDA-HRV | EDA, PPG | handcrafted | low | no | EDA features: Mean Tonic Component, SD Tonic (Standard deviation of the tonic component), Perc20 (20th percentile of the phasic component), Perc80 Tonic (80th percentile of the tonic component), Quartdev Tonic (Quartile deviation 75 percentile - 25 percentile of the tonic component), Strong Peaks (The number of strong peaks per 100 seconds) and Peaks Phasic (The number of peaks per 100 seconds). HRV features: Time Domain Features: Mean HR, standard deviation of the inter-beat interval (STD RR), mean value of the inter-beat (RR) intervals (Mean RR), root mean square of successive difference of the RR intervals (RMSSD), the percentage of the number of successive RR intervals varying more than 50ms from the previous interval (pNN50), the total number of RR intervals divided by the height of the histogram of all RR intervals measured on a scale with bins of 1/128 s (HRV triangular index), and triangular interpolation of RR interval histogram (TINN). Frequency Domain Features: Low frequency power (LF), high-frequency power (HF), very low frequency power (VLF), prevalent low frequency (pLF), prevalent high frequency (pHF), the ratio of LF to HF (LF/HF) (Preprocessing +FFT), LF, HF, LF/HF (Lomb-Scargle) |
| Other | 305 | MLP 120 EDA | EDA | handcrafted | low | no | EDA features: Mean Tonic Component, SD Tonic (Standard deviation of the tonic component), Perc20 (20th percentile of the phasic component), Perc80 Tonic (80th percentile of the tonic component), Quartdev Tonic (Quartile deviation 75 percentile - 25 percentile of the tonic component), Strong Peaks (The number of strong peaks per 100 seconds) and Peaks Phasic (The number of peaks per 100 seconds). |
| Other | 305 | MLP 120 EDA-HRV | EDA, PPG | handcrafted | low | no | EDA features: Mean Tonic Component, SD Tonic (Standard deviation of the tonic component), Perc20 (20th percentile of the phasic component), Perc80 Tonic (80th percentile of the tonic component), Quartdev Tonic (Quartile deviation 75 percentile - 25 percentile of the tonic component), Strong Peaks (The number of strong peaks per 100 seconds) and Peaks Phasic (The number of peaks per 100 seconds). HRV features: Time Domain Features: Mean HR, standard deviation of the inter-beat interval (STD RR), mean value of the inter-beat (RR) intervals (Mean RR), root mean square of successive difference of the RR intervals (RMSSD), the percentage of the number of successive RR intervals varying more than 50ms from the previous interval (pNN50), the total number of RR intervals divided by the height of the histogram of all RR intervals measured on a scale with bins of 1/128 s (HRV triangular index), and triangular interpolation of RR interval histogram (TINN). Frequency Domain Features: Low frequency power (LF), high-frequency power (HF), very low frequency power (VLF), prevalent low frequency (pLF), prevalent high frequency (pHF), the ratio of LF to HF (LF/HF) (Preprocessing +FFT), LF, HF, LF/HF (Lomb-Scargle) |
| Other | 305 | MLP 30 EDA | EDA | handcrafted | low | no | EDA features: Mean Tonic Component, SD Tonic (Standard deviation of the tonic component), Perc20 (20th percentile of the phasic component), Perc80 Tonic (80th percentile of the tonic component), Quartdev Tonic (Quartile deviation 75 percentile - 25 percentile of the tonic component), Strong Peaks (The number of strong peaks per 100 seconds) and Peaks Phasic (The number of peaks per 100 seconds). |
| Other | 305 | MLP 30 EDA-HRV | EDA, PPG | handcrafted | low | no | EDA features: Mean Tonic Component, SD Tonic (Standard deviation of the tonic component), Perc20 (20th percentile of the phasic component), Perc80 Tonic (80th percentile of the tonic component), Quartdev Tonic (Quartile deviation 75 percentile - 25 percentile of the tonic component), Strong Peaks (The number of strong peaks per 100 seconds) and Peaks Phasic (The number of peaks per 100 seconds). HRV features: Time Domain Features: Mean HR, standard deviation of the inter-beat interval (STD RR), mean value of the inter-beat (RR) intervals (Mean RR), root mean square of successive difference of the RR intervals (RMSSD), the percentage of the number of successive RR intervals varying more than 50ms from the previous interval (pNN50), the total number of RR intervals divided by the height of the histogram of all RR intervals measured on a scale with bins of 1/128 s (HRV triangular index), and triangular interpolation of RR interval histogram (TINN). Frequency Domain Features: Low frequency power (LF), high-frequency power (HF), very low frequency power (VLF), prevalent low frequency (pLF), prevalent high frequency (pHF), the ratio of LF to HF (LF/HF) (Preprocessing +FFT), LF, HF, LF/HF (Lomb-Scargle) |
| Other | 305 | MLP 60 EDA | EDA | handcrafted | low | no | EDA features: Mean Tonic Component, SD Tonic (Standard deviation of the tonic component), Perc20 (20th percentile of the phasic component), Perc80 Tonic (80th percentile of the tonic component), Quartdev Tonic (Quartile deviation 75 percentile - 25 percentile of the tonic component), Strong Peaks (The number of strong peaks per 100 seconds) and Peaks Phasic (The number of peaks per 100 seconds). |
| Other | 305 | MLP 60 EDA-HRV | EDA, PPG | handcrafted | low | no | EDA features: Mean Tonic Component, SD Tonic (Standard deviation of the tonic component), Perc20 (20th percentile of the phasic component), Perc80 Tonic (80th percentile of the tonic component), Quartdev Tonic (Quartile deviation 75 percentile - 25 percentile of the tonic component), Strong Peaks (The number of strong peaks per 100 seconds) and Peaks Phasic (The number of peaks per 100 seconds). HRV features: Time Domain Features: Mean HR, standard deviation of the inter-beat interval (STD RR), mean value of the inter-beat (RR) intervals (Mean RR), root mean square of successive difference of the RR intervals (RMSSD), the percentage of the number of successive RR intervals varying more than 50ms from the previous interval (pNN50), the total number of RR intervals divided by the height of the histogram of all RR intervals measured on a scale with bins of 1/128 s (HRV triangular index), and triangular interpolation of RR interval histogram (TINN). Frequency Domain Features: Low frequency power (LF), high-frequency power (HF), very low frequency power (VLF), prevalent low frequency (pLF), prevalent high frequency (pHF), the ratio of LF to HF (LF/HF) (Preprocessing +FFT), LF, HF, LF/HF (Lomb-Scargle) |
| Other | 305 | RF 120 EDA | EDA | handcrafted | low | no | EDA features: Mean Tonic Component, SD Tonic (Standard deviation of the tonic component), Perc20 (20th percentile of the phasic component), Perc80 Tonic (80th percentile of the tonic component), Quartdev Tonic (Quartile deviation 75 percentile - 25 percentile of the tonic component), Strong Peaks (The number of strong peaks per 100 seconds) and Peaks Phasic (The number of peaks per 100 seconds). |
| Other | 305 | RF 120 EDA-HRV | EDA, PPG | handcrafted | low | no | EDA features: Mean Tonic Component, SD Tonic (Standard deviation of the tonic component), Perc20 (20th percentile of the phasic component), Perc80 Tonic (80th percentile of the tonic component), Quartdev Tonic (Quartile deviation 75 percentile - 25 percentile of the tonic component), Strong Peaks (The number of strong peaks per 100 seconds) and Peaks Phasic (The number of peaks per 100 seconds). HRV features: Time Domain Features: Mean HR, standard deviation of the inter-beat interval (STD RR), mean value of the inter-beat (RR) intervals (Mean RR), root mean square of successive difference of the RR intervals (RMSSD), the percentage of the number of successive RR intervals varying more than 50ms from the previous interval (pNN50), the total number of RR intervals divided by the height of the histogram of all RR intervals measured on a scale with bins of 1/128 s (HRV triangular index), and triangular interpolation of RR interval histogram (TINN). Frequency Domain Features: Low frequency power (LF), high-frequency power (HF), very low frequency power (VLF), prevalent low frequency (pLF), prevalent high frequency (pHF), the ratio of LF to HF (LF/HF) (Preprocessing +FFT), LF, HF, LF/HF (Lomb-Scargle) |
| Other | 305 | RF 30 EDA | EDA | handcrafted | low | no | EDA features: Mean Tonic Component, SD Tonic (Standard deviation of the tonic component), Perc20 (20th percentile of the phasic component), Perc80 Tonic (80th percentile of the tonic component), Quartdev Tonic (Quartile deviation 75 percentile - 25 percentile of the tonic component), Strong Peaks (The number of strong peaks per 100 seconds) and Peaks Phasic (The number of peaks per 100 seconds). |
| Other | 305 | RF 30 EDA-HRV | EDA, PPG | handcrafted | low | no | EDA features: Mean Tonic Component, SD Tonic (Standard deviation of the tonic component), Perc20 (20th percentile of the phasic component), Perc80 Tonic (80th percentile of the tonic component), Quartdev Tonic (Quartile deviation 75 percentile - 25 percentile of the tonic component), Strong Peaks (The number of strong peaks per 100 seconds) and Peaks Phasic (The number of peaks per 100 seconds). HRV features: Time Domain Features: Mean HR, standard deviation of the inter-beat interval (STD RR), mean value of the inter-beat (RR) intervals (Mean RR), root mean square of successive difference of the RR intervals (RMSSD), the percentage of the number of successive RR intervals varying more than 50ms from the previous interval (pNN50), the total number of RR intervals divided by the height of the histogram of all RR intervals measured on a scale with bins of 1/128 s (HRV triangular index), and triangular interpolation of RR interval histogram (TINN). Frequency Domain Features: Low frequency power (LF), high-frequency power (HF), very low frequency power (VLF), prevalent low frequency (pLF), prevalent high frequency (pHF), the ratio of LF to HF (LF/HF) (Preprocessing +FFT), LF, HF, LF/HF (Lomb-Scargle) |
| Other | 305 | RF 60 EDA | EDA | handcrafted | low | no | EDA features: Mean Tonic Component, SD Tonic (Standard deviation of the tonic component), Perc20 (20th percentile of the phasic component), Perc80 Tonic (80th percentile of the tonic component), Quartdev Tonic (Quartile deviation 75 percentile - 25 percentile of the tonic component), Strong Peaks (The number of strong peaks per 100 seconds) and Peaks Phasic (The number of peaks per 100 seconds). |
| Other | 305 | RF 60 EDA-HRV | EDA, PPG | handcrafted | low | no | EDA features: Mean Tonic Component, SD Tonic (Standard deviation of the tonic component), Perc20 (20th percentile of the phasic component), Perc80 Tonic (80th percentile of the tonic component), Quartdev Tonic (Quartile deviation 75 percentile - 25 percentile of the tonic component), Strong Peaks (The number of strong peaks per 100 seconds) and Peaks Phasic (The number of peaks per 100 seconds). HRV features: Time Domain Features: Mean HR, standard deviation of the inter-beat interval (STD RR), mean value of the inter-beat (RR) intervals (Mean RR), root mean square of successive difference of the RR intervals (RMSSD), the percentage of the number of successive RR intervals varying more than 50ms from the previous interval (pNN50), the total number of RR intervals divided by the height of the histogram of all RR intervals measured on a scale with bins of 1/128 s (HRV triangular index), and triangular interpolation of RR interval histogram (TINN). Frequency Domain Features: Low frequency power (LF), high-frequency power (HF), very low frequency power (VLF), prevalent low frequency (pLF), prevalent high frequency (pHF), the ratio of LF to HF (LF/HF) (Preprocessing +FFT), LF, HF, LF/HF (Lomb-Scargle) |
| Other | 382 | 382.0.1 HRB CNN | PPG, microphone | hybrid | medium | no | Statistical Features (mean, standard deviation, variance, coefficient of variance, range, coefficient of range, energy, power, root mean square, first quartile, second quartile or median, third quartile, maximum, skewness, interquartile range, coefficient of interquartile, mean absolute deviation, kurtosis, median absolute deviation, root sum of squares, and signal to noise ratio) + spectral features (Two-dimensional Mel-frequency cepstral coefficient features are transformed into one-dimensional features by condensing along the time axis using the average values) |
| Other | 382 | 382.0.1 HRB CRNN | PPG, microphone | hybrid | medium | no | Statistical Features (mean, standard deviation, variance, coefficient of variance, range, coefficient of range, energy, power, root mean square, first quartile, second quartile or median, third quartile, maximum, skewness, interquartile range, coefficient of interquartile, mean absolute deviation, kurtosis, median absolute deviation, root sum of squares, and signal to noise ratio) + spectral features (Two-dimensional Mel-frequency cepstral coefficient features are transformed into one-dimensional features by condensing along the time axis using the average values) |
| Other | 382 | 382.0.1 HRB RNN | PPG, microphone | hybrid | medium | no | Statistical Features (mean, standard deviation, variance, coefficient of variance, range, coefficient of range, energy, power, root mean square, first quartile, second quartile or median, third quartile, maximum, skewness, interquartile range, coefficient of interquartile, mean absolute deviation, kurtosis, median absolute deviation, root sum of squares, and signal to noise ratio) + spectral features (Two-dimensional Mel-frequency cepstral coefficient features are transformed into one-dimensional features by condensing along the time axis using the average values) |
| Other | 61 | L3aHR+Resp binary SVM | PPG+microphone | handcrafted | low | yes | HRV: 21 statistical features: mean, median, standard deviation, variance, coef cient of variance, range, coefficient of range, first quartile or 25th percentile, third quartile or 75th percentile, maximum, interquartile range, coefficient of interquartile, mean absolute deviation, median absolute deviation, energy, power, root mean square, root sum of squares, signal to noise ratio, skewness, and kurtosis; Resp: 182 features, 40 Mel-frequency cepstral coefficients (MFCCs), 13 Gammatone frequency cepstral coef cients (GFCCs), 13 Bark frequency cepstral coef cients (BFCCs), 13 Linear predictive coding (LPC), 13 Revised perceptual linear prediction coef cients (RPLP), 12 chromas, and six tonal features. 5 statistical features from each chroma feature and tonal feature. |
| Other | 61 | L3aHR+Resp unary SVM | PPG+microphone | handcrafted | medium | no | HRV: 21 statistical features: mean, median, standard deviation, variance, coef cient of variance, range, coefficient of range, first quartile or 25th percentile, third quartile or 75th percentile, maximum, interquartile range, coefficient of interquartile, mean absolute deviation, median absolute deviation, energy, power, root mean square, root sum of squares, signal to noise ratio, skewness, and kurtosis; Resp: 182 features, 40 Mel-frequency cepstral coefficients (MFCCs), 13 Gammatone frequency cepstral coef cients (GFCCs), 13 Bark frequency cepstral coef cients (BFCCs), 13 Linear predictive coding (LPC), 13 Revised perceptual linear prediction coef cients (RPLP), 12 chromas, and six tonal features. 5 statistical features from each chroma feature and tonal feature. |
| Other | 75 | 1D CNN rr | VHF | handcrafted | medium | yes | autocorrelation (AC), discrete cosine transform (DCT), and the mel-frequency cepstrum coefficient (MFCC); (20 AC/DCT and 24 MFCC features) from I and Q waveforms |
| Other | 75 | 1D CNN rw | VHF | handcrafted | medium | yes | autocorrelation (AC), discrete cosine transform (DCT), and the mel-frequency cepstrum coefficient (MFCC); (20 AC/DCT and 24 MFCC features) from I and Q waveforms |
| Other | 75 | 1D CNN ww | VHF | handcrafted | medium | yes | autocorrelation (AC), discrete cosine transform (DCT), and the mel-frequency cepstrum coefficient (MFCC); (20 AC/DCT and 24 MFCC features) from I and Q waveforms |
| Other | 75 | MLP AC/DCT | VHF | handcrafted | medium | yes | 20 autocorrelation (AC) and discrete cosine transform (DCT) features from I and Q waveforms |
| Other | 75 | MLP AC/DCT+MFCC | VHF | handcrafted | medium | yes | autocorrelation (AC), discrete cosine transform (DCT), and the mel-frequency cepstrum coefficient (MFCC); (20 AC/DCT and 24 MFCC features) from I and Q waveforms |
| Other | 75 | MLP MFCC | VHF | handcrafted | medium | yes | 24 mel-frequency cepstrum coefficient (MFCC) features from I and Q waveforms |
| Other | 75 | MLP rr | VHF | handcrafted | medium | yes | autocorrelation (AC), discrete cosine transform (DCT), and the mel-frequency cepstrum coefficient (MFCC); (20 AC/DCT and 24 MFCC features) from I and Q waveforms |
| Other | 75 | MLP rw | VHF | handcrafted | medium | yes | autocorrelation (AC), discrete cosine transform (DCT), and the mel-frequency cepstrum coefficient (MFCC); (20 AC/DCT and 24 MFCC features) from I and Q waveforms |
| Other | 75 | MLP ww | VHF | handcrafted | medium | yes | autocorrelation (AC), discrete cosine transform (DCT), and the mel-frequency cepstrum coefficient (MFCC); (20 AC/DCT and 24 MFCC features) from I and Q waveforms |
| Other | 75 | RF AC/DCT | VHF | handcrafted | medium | yes | 20 autocorrelation (AC) and discrete cosine transform (DCT) features from I and Q waveforms |
| Other | 75 | RF AC/DCT+MFCC | VHF | handcrafted | medium | yes | autocorrelation (AC), discrete cosine transform (DCT), and the mel-frequency cepstrum coefficient (MFCC); (20 AC/DCT and 24 MFCC features) from I and Q waveforms |
| Other | 75 | RF MFCC | VHF | handcrafted | medium | yes | 24 mel-frequency cepstrum coefficient (MFCC) features from I and Q waveforms |
| Other | 75 | RF rr | VHF | handcrafted | medium | yes | autocorrelation (AC), discrete cosine transform (DCT), and the mel-frequency cepstrum coefficient (MFCC); (20 AC/DCT and 24 MFCC features) from I and Q waveforms |
| Other | 75 | RF rw | VHF | handcrafted | medium | yes | autocorrelation (AC), discrete cosine transform (DCT), and the mel-frequency cepstrum coefficient (MFCC); (20 AC/DCT and 24 MFCC features) from I and Q waveforms |
| Other | 75 | RF ww | VHF | handcrafted | medium | yes | autocorrelation (AC), discrete cosine transform (DCT), and the mel-frequency cepstrum coefficient (MFCC); (20 AC/DCT and 24 MFCC features) from I and Q waveforms |
| Other | 75 | SVM AC/DCT | VHF | handcrafted | medium | yes | 20 autocorrelation (AC) and discrete cosine transform (DCT) features from I and Q waveforms |
| Other | 75 | SVM AC/DCT+MFCC | VHF | handcrafted | medium | yes | autocorrelation (AC), discrete cosine transform (DCT), and the mel-frequency cepstrum coefficient (MFCC); (20 AC/DCT and 24 MFCC features) from I and Q waveforms |
| Other | 75 | SVM MFCC | VHF | handcrafted | medium | yes | 24 mel-frequency cepstrum coefficient (MFCC) features from I and Q waveforms |
| Other | 75 | SVM rr | VHF | handcrafted | medium | yes | autocorrelation (AC), discrete cosine transform (DCT), and the mel-frequency cepstrum coefficient (MFCC); (20 AC/DCT and 24 MFCC features) from I and Q waveforms |
| Other | 75 | SVM rw | VHF | handcrafted | medium | yes | autocorrelation (AC), discrete cosine transform (DCT), and the mel-frequency cepstrum coefficient (MFCC); (20 AC/DCT and 24 MFCC features) from I and Q waveforms |
| Other | 75 | SVM ww | VHF | handcrafted | medium | yes | autocorrelation (AC), discrete cosine transform (DCT), and the mel-frequency cepstrum coefficient (MFCC); (20 AC/DCT and 24 MFCC features) from I and Q waveforms |
| Other | 75 | XGB AC/DCT | VHF | handcrafted | medium | yes | 20 autocorrelation (AC) and discrete cosine transform (DCT) features from I and Q waveforms |
| Other | 75 | XGB AC/DCT+MFCC | VHF | handcrafted | medium | yes | autocorrelation (AC), discrete cosine transform (DCT), and the mel-frequency cepstrum coefficient (MFCC); (20 AC/DCT and 24 MFCC features) from I and Q waveforms |
| Other | 75 | XGB MFCC | VHF | handcrafted | medium | yes | 24 mel-frequency cepstrum coefficient (MFCC) features from I and Q waveforms |
| Other | 75 | XGB rr | VHF | handcrafted | medium | yes | autocorrelation (AC), discrete cosine transform (DCT), and the mel-frequency cepstrum coefficient (MFCC); (20 AC/DCT and 24 MFCC features) from I and Q waveforms |
| Other | 75 | XGB rw | VHF | handcrafted | medium | yes | autocorrelation (AC), discrete cosine transform (DCT), and the mel-frequency cepstrum coefficient (MFCC); (20 AC/DCT and 24 MFCC features) from I and Q waveforms |
| Other | 75 | XGB ww | VHF | handcrafted | medium | yes | autocorrelation (AC), discrete cosine transform (DCT), and the mel-frequency cepstrum coefficient (MFCC); (20 AC/DCT and 24 MFCC features) from I and Q waveforms |